80W LED Beam Moving Head Light

USER MANUAL

KEEP THIS MANUAL FOR FUTURE NEEDS

1.   General Information:

Thank you very much for choosing our product. For safety purpose, please read this manual carefully before your operation. This manual included installation and using information. Please install and operate it according to this manual.

Unpacking:Once receive it,please handle it carefully and check if there is some shatter because of transportation and the following fittings are OK.

In the carton,there are four things available:

Moving head light: 1PC

Power cable: 1PC

Fast Lock: 2PC

User manual: 1 PC

Warning! To prevent or reduce the risk of electrical shock or fire, do not expose this unit to rain or moisture.

Caution! There are no user serviceable parts inside this unit. Do not attempt any repairs yourself. Doing so will void your manufactures warranty. In the unlikely event your unit may require service please contact us.

2.   Safety Instructions

- Please keep this User Guide for future consultation. If you sell the unit to another user, be sure that they also receive this instruction booklet.

- Unpack and check carefully there is no transportation damage before using the unit.

- Before operating, ensure that the voltage and frequency of power supply match the power requirements of the unit.

- It’s important to ground the yellow/green conductor to earth in order to avoid electric shock.

- The unit is for indoor use only. Use only in a dry location.

- The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent surfaces. Be sure that no ventilation slots are blocked.

- Disconnect main power before replacement or servicing.

- Make sure there are no flammable materials close to the unit while operating as it is fire hazard.

- Use safety cable when fixes this unit. DO NOT handle the unit by taking its head only, but always by taking its base.

- Maximum ambient temperature is Ta: 40℃. DO NOT operate it where the temperature is higher than this.

- Unit surface temperature may reach up to 85℃. DO NOT touch the housing bare-hand during its operation. Turn off the power and allow about 15 minutes for the unit to cool down before replacing or serving.

- In the event of serious operating problem, stop using the unit immediately. Never try to repair the unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please contact the nearest authorized technical assistance center. Always use the same type spare parts.

- DO NOT touch any wire during operation as high voltage might be causing electric shock.

Warning:

- To prevent or reduce the risk of electrical shock or fire, do not expose the unit to rain or moisture.

- DO NOT open the unit within five minutes after switching off.

- The housing, the lenses, or the ultraviolet filter must be replaced if they are visibly damaged.

Caution:

There are no user serviceable parts inside the unit. DO NOT open the housing or attempt any repairs yourself. In the unlikely event your unit may require service, please contact your nearest dealer.

Installation:

The unit should be mounted via its screw holes on the bracket. Always ensure that the unit is firmly fixed to avoid vibration and slipping while operating. And make sure that the structure to which you are attaching the unit is secure and is able to support a weight of 10 times of the unit’s weight. Also always use a safety cable that can hold 12 times of the weight of the unit when installing the fixture.

The equipment must be fixed by professionals. And it must be fixed at a place where is out of the touch of people and has no one pass by or under it.

3.   Technical Specifications

Voltage: AC100-240V, 50/60Hz

Power: 120W

LED Source: 1pc High power 80W White LED@4500lm

Over 50000hrs working life

Color Temperature: 7000k-7500k

Beam Angle: 2degree

Lumin: 123000lux @ 4m

DMX Channel:  16CH/20CH

Control Mode: DMX 512, Master/Slave, Auto, Sound active

Support RDM

Movement: Pan: 540° / Tilt: 270 °, 16bit Fine Pan/Tilt control

Color Wheel:  9colors + white, half-color, rainbow effect

Gobo:  15 fixed gobo+blank, Gobo shake

7Colors Effect: An independent wheel with 6colors , can make the gobos in rainbow effect

Frost: A frost soft light mirror

Dimmer: 0-100% linear adjustment

Strobe: Synchronized Pulse strobe, Random pulse strobe

Prism: Double Prisms, 8 facet rotate prism + 24 facet rotate prism, double prism can Overlapped

Focus: 0-100% Linear adjustment focus

IP Rate: IP20

Display: LCD display, Chinese & English surface, reverse 180º displayed

High efficiency cooling system, Temperature monitoring, Over temperature automatic protection

N.W: 11.88kgs

Size: 280*215*510mm

- Photometric Diagram:

5.  Interface Description

Main Interface

Chinese/English surface

Reverse 180 displayed

DMX Address

System Information

Setting

Manual

6.  Subinterface

Manual Control

(2) Press “OK” key enter into Edit status, the word is red, then press “UP” “DOWN” to select, press “OK” save and Exit edit status.

(1)Yellow arrow indicates selected

This interface is used to control the current luminaire and automatically enters the host state (does not receive the DMX signal and sends a DMX signal to the slave to the host).

The manual menu will display 16 channels or 20 channels according to the standard 16-channel or extended 20-channel mode set in the setup menu.

| Option | Description |  |
| --- | --- | --- |
| 1CH. Pan | 0-255 | Yellow arrow indicates selected, Press “OK” key enter into Edit status, the word is red, then press “UP” “DOWN” to select, press “OK” save and Exit edit status. |
| …… | 0-255 |  |
| 15CH. Frost | 0-255 |  |
| 16CH. Reset |  | Press “OK” to see the confirmation dialog box, press “OK” again to enter the reset interface, all motor reset |
| 17CH. Pan/Tilt Speed | 0-255 | Channel Mode is”Extended 20-channel” can display |
| 18CH. Color Wheel Speed | 0-255 |  |
| 19CH.Dimmer-Prism-Frost Speed | 0-255 |  |
| 20CH. Gobo Wheel Speed | 0-255 |  |

Setting

(2) Press “OK” key enter into Edit status, the word is red, then press “UP” “DOWN” to select, press “OK” save and Exit edit status.

(1)Yellow arrow indicates selected

| Option | Description |  |
| --- | --- | --- |
| Run Mode | DMX | Slave status: Receive DMX signals from the console or Master |
|  | AUTO 1 | Auto run ; Master status: Auto run and send DMX signal to slave |
|  | AUTO 2 |  |
|  | Random |  |
|  | Sound |  |
| DMX Address | 001-512 | Press “OK” key enter into edit status, then press “UP” “DOWN” to change DMX address. press “OK” key save and exit edit status. |
| Channel Mode | 16 CH | Standard 16 Channel Mode, 17-20CH No Function |
|  | 20 CH | Extend 20 Channel Mode, 17-20CH for speed control. (please check the detail on Channel table.) |
| Invert Pan | OFF | ON for starting and ending , the default is OFF. |
|  | ON |  |
| Invert Tilt | OFF | ON for starting and ending , the default is OFF. |
|  | ON |  |
| Pan-Tilt Swap | OFF |  |
|  | ON | Exchange Pan/Tilt channel (include Pan/Tilt Fine) |
| Pan-Tilt Encoder | ON | Use encoder (optocoupler) to determine out of step and automatically correct position |
|  | OFF | Correct position without encoder (optocoupler) |
| DMX Signal | KEEP | Continue running as it is |
|  | CLEAR | All motors return to the position and stop running. |
| Linear Color | ON | Color liner change |
|  | OFF | Color nonlinear change, half color change |
| Load Default |  | Press the "OK" button to see the confirmation dialog, press "OK" again The key restores the default settings |

System

Version No.

| Option | Description |  |
| --- | --- | --- |
| Version No.(V) |  | Version No. |
| Reset Calibration | Pan Calibration | After entering the sub-interface, you can adjust the motor such as X-axis and Y-axis. Reset position to compensate for errors in hardware installation, Adjustment range -128~+127, +0 means no adjustment. |
|  | Tilt Calibration |  |
|  | Color Calibration |  |
|  | Gobo Calibration |  |
|  | Focus Calibration |  |
|  | Prism1 Calibration |  |
|  | Prism2 Calibration |  |
|  | Rainbow Calibration |  |
|  | Frost Calibration |  |
| Sensor Monitor | Pan Hall | 0 when magnetic is detected, otherwise 1 |
|  | Tilt Hall |  |
|  | Color Hall |  |
|  | Gobo Hall |  |
|  | Focus Hall |  |
|  | Prism1 Hall |  |
|  | Prism2 Hall |  |
|  | Panwheel State | 2 digits, each corresponding to a photoelectric switch in the encoder disc |
|  | Tiltwheel State |  |
|  | Panwheel Step | When walking in the positive direction, the step value should increase, when going back, The step value should be reduced. The value is the same every time you go to a certain point |
|  | Tiltwheel Step |  |
| System Error | (See common mistakes Rear) | If the red ERR indicator is lit, the luminaire is running. An error can be made by going to the sub-interface for details. After viewing, you can press the "Clear" button to clear the error record. |
| DMX Monitor |  | This enters the sub-interface and displays the channel values as values for viewing. |

| Error | Description |
| --- | --- |
| MT board connection failed | The driver board did not respond. Serial communication line connecting display panel and driver board There is a problem, or there is a problem with the driver board. |
| Pan Reset failed | X-axis photoelectric switch, or problem with X-axis motor |
| Tilt Reset failed | Y-axis photoelectric switch, or problem with Y-axis motor |
| Pan Hall failed | Pan Hall have problem |
| Tilt Hall failed | Tilt Hall have problem |
| Color Wheel Reset failed | Color Wheel Hall , or Color Wheel motor have problem |
| Gobo Wheel Reset failed | Gobo Wheel Hall , or Gobo Wheel motor have problem |
| Focus Reset failed | Focus Hall , or Focus motor have problem |

7.  DMX Channel Table

| CH | Channel Mode |  |
| --- | --- | --- |
|  | 16CH | 20CH |
| 1 | Pan | Pan |
| 2 | Pan Fine | Pan Fine |
| 3 | Tilt | Tilt |
| 4 | Tilt Fine | Tilt Fine |
| 5 | Dimmer | Dimmer |
| 6 | Strobe/Shutter | Strobe/Shutter |
| 7 | Color Wheel | Color Wheel |
| 8 | Gobo Wheel | Gobo Wheel |
| 9 | Prism 1 | Prism 1 |
| 10 | Prism 1 Rotation | Prism 1 Rotation |
| 11 | Prism 2 | Prism 2 |
| 12 | Prism 2 Rotation | Prism 2 Rotation |
| 13 | Focus | Focus |
| 14 | 7-Color Effect | 7-Color Effect |
| 15 | Frost | Frost |
| 16 | Reset | Reset |
| 17 |  | Pan/Tilt Speed |
| 18 |  | Color Wheel Speed |
| 19 |  | Dimmer-Prism-Frost Speed |
| 20 |  | Gobo Wheel Speed |

8.  16/20 DMX Channel Description

| CH | Function | Value | Effect |
| --- | --- | --- | --- |
| 1 | Pan | 000-255 | Pan 540 degree scan |
| 2 | Pan Fine | 000-255 | Pan 1.2 degree scan |
| 3 | Tilt | 000-255 | Tilt 270 degree scan |
| 4 | Tilt Fine | 000-255 | Tilt 1.2 degree scan |
| 5 | Dimmer | 000-255 | From dark to bright |
| 6 | Strobe | 000-003 004-251 252-255 | OFF Strobe from slow to fast ON→(control by dimmer channel) |
| 7 | Color Wheel | 000-004 005 -009 010 – 014 015 – 019 020 – 024 025 – 029 030 – 034 035 – 039 040 – 044 045 – 049 050 – 054 055 – 059 060 – 064 065 – 069 070 – 074 075 – 079 080 – 084 085 – 089 090 – 094 095 – 099 100 -177 178 - 255 | White White + Color1 Color 1 Color 1 + Color 2 Color 2 Color 2 + Color 3 Color 3 Color 3 + Color 4 Color 4 Color 4 + Color 5 Color 5 Color 5 + Color 6 Color 6 Color 6 + Color 7 Color 7 Color 7 + Color 8 Color 8 Color 8 + Color 9 Color 9 Color 9 + White Positive flow (from fast to slow) Reverse flow (from slow to fast) |
| 8 | Gobo Wheel | 000 – 004 005 – 009 010 – 014 015 – 019 020 – 024 025 – 029 030 – 034 035 – 039 040 – 044 045 – 049 050 – 054 055 – 059 060 – 064 065 – 069 070 – 074 075 – 079 080 – 084 085 – 089 090 – 094 095 – 099 100 – 104 105 – 109 110 – 114 115 – 119 120 – 124 125 – 129 130 – 134 135 – 139 140 – 144 145 – 149 150 – 154 155 – 159 160 – 207 208 – 255 | White Gobo 1 Gobo 2 Gobo 3 Gobo 4 Gobo 5 Gobo 6 Gobo 7 Gobo 8 Gobo 9 Gobo 10 Gobo 11 Gobo 12 Gobo 13 Gobo 14 Gobo 15 White shake (from slow to fast) Gobo 1 shake (from slow to fast) Gobo 2 shake (from slow to fast) Gobo 3 shake (from slow to fast) Gobo 4 shake (from slow to fast) Gobo 5 shake (from slow to fast) Gobo 6 shake (from slow to fast) Gobo 7 shake (from slow to fast) Gobo 8 shake (from slow to fast) Gobo 9 shake (from slow to fast) Gobo 10 shake (from slow to fast) Gobo 11 shake (from slow to fast) Gobo 12 shake (from slow to fast) Gobo 13 shake (from slow to fast) Gobo 14 shake (from slow to fast) Gobo 15 shake (from slow to fast) Positive flow (from fast to slow) Reverse flow (from slow to fast) |
| 9 | Prism 1 | 000-127 128-255 | Prism 1 out Prism 1 in |
| 10 | Prism 1 Rotation | 000-127 128-190 191-192 193-255 | Prism angle adjustment Reverse rotation (from fast to slow) Stop Forward rotation (from slow to fast) |
| 11 | Prism 2 | 000-127 128-255 | Prism 2 out Prism 2 in |
| 12 | Prism 2 Rotation | 000-127 128-190 191-192 193-255 | Prism angle adjustment Reverse rotation (from fast to slow) Stop Forward rotation (from slow to fast) |
| 13 | Focus | 000-255 | Pattern definition from far to near |
| 14 | 7-Color Effect | 000-127 128-255 | 7-Color Effect out 7-Color Effect in |
| 15 | Frost | 000-127 128-255 | Frost out Frost in |
| 16 | Reset | 000-049 050-099 100-199 200-255 | No Function Small Motor Reset Big Motor (Pan/Tilt) Reset All Motor Reset |
| Extend Channel: |  |  |  |
| 17 | Pan/Tilt Speed | 000-255 | Speed from fast to slow |
| 18 | Color Wheel Speed |  |  |
| 19 | Dimmer-Prism-Frost Speed |  |  |
| 20 | Gobo Wheel Speed |  |  |

9.  Special Instructions

- During the reset process, press and hold the 5 seconds “OK” button to interrupt the reset.

- Press and hold the “OK” key during power-on to interrupt the reset process and enter test mode.

- Set the DMX address to 512, return to the main interface, press and hold the 10 seconds “OK” button to set the “display”.Or "hidden" the LOGO.

- Signal indicator:

- The ERR red indicator flashes to indicate that there is an error message and enters the "System Information" -> System Error" view information.

- DMX green indicator light, steady light indicates that the DMX signal is received, and frequent off indicates no DMX signal.

- The green indicator on the motor drive board flashes quickly at 1 second intervals, indicating that it is received.The serial port signal sent by the display board; if it flashes slowly at 2 second intervals, it means no string Port signal, the flashing of the light is used to indicate that the system is running; if the indicator light is always on or off, it means There is a problem with the motor drive board.