The LED digital display of the PAR lamp use instructions

- Graphic section:

A. function key

B. additional key

C. The key reduction

D. Confirmation key

E.LED display window

A    B    C   D

Operation description: Press function key A will cycle ten different function effects. The first two digits of digital pipe represent the current function (refer to the function table). The latter two numbers represent where the function is located.address code. Or the parameters of the speed, or of the. Press either B or C to modify its parameter value. Press the D key to confirm.

- LED display window function (all functions are selected and press D key to confirm)

| order number | show | function declaration |
| --- | --- | --- |
| 1 | A 001 | 8-Channel address code, (001- -512) Key B and C keys add and subtract the address code value |
| 2 | d 001 | 4-Channel address code, (001- -512) Key B and C keys add and subtract the address code value |
| 3 | CC 99 | Colorful jump change, (00- -99) B, C keys to modify the colorful jump speed |
| 4 | CP99 | Colorful gradient change, (00- -99) B, C keys to modify the seven-color gradient speed |
| 5 | d E99 | Colorful vein change, (00- -99) - Key C changes the pulse speed |
| 6 | SOUd | Colorful voice control changes |
| 7 | R255 | Red color selection, (000- -255) B, C keys to modify the color selection |
| 8 | G255 | Green color selection, (000- -255) - C key modifies color selection |
| 9 | B255 | Blue color selection, (000- -255) - C key modifies color selection |
| 10 | O 255 | White Color Selection, (000- -255) - C key modifies color selection |

- LED background display window function control table, (in the A001 state, often press A key for 3-4 seconds to enter and press D key to exit)

| order number | show | function declaration |
| --- | --- | --- |
| 1 | R235 | Red current, (020- -255) Default value (235) |
| 2 | G235 | Green current, (020- -255) default value (235) |
| 3 | B235 | Blue color current, (020-255) default value (235) |
| 4 | O 235 | White current, (020- -255) Default value (235) |
| 5 | n255 | Total power limit, (050-255) default value (255 no limit) |
| 6 | LEdF | LEdF (host background setting parameter) does not push the default value to the slave (LEdF) |
|  |  | The LEdO host pushes to the slave to update the slave background parameters Note: The signal line between the host and the slave is connected and connected, the slave is set to DMX512 mode, the host does not rush to exit the slave lamp bead flashing means that the slave update is successful, then the host press the confirmation key to exit. |
| 7 | LoAd | LoAd does not restore the factory setting defaults (LoAd) |
|  |  | LdEd restores factory settings Note: LdEd flashes whether to enter LdEd. After entering LdEd, the menu does not loop LdEd to save the exit L d E d. If you do not need to restore the factory settings, press B or C in LdEd mode to turn the LoAd back to the menu in LoAd mode. Restore factory settings (current value) and (total power limit) will not be restored. |
| 8 | C-55 | Light body temperature value setting, (40-70) default value (55) Note: the light body should be connected to the temperature control resistance before this function works |
| 9 | d o n- | d o FF dmx512 segment signal is not maintained (light enters closed light after segment signal) |
|  |  | The d o n-dmx512 segment signal hold (segment signal back light hold segment signal front state) Default value (d o n-) |
| 10 | E o n- | The E o FF display is always backlit |
|  |  | E o n-Display default default (E o n-) |
| 11 | FAN1 | FAN1 fan temperature control and light control default value (FAN1) Note: when the temperature display is above 45 degrees and the fan is on, the temperature display is low 40 degrees regardless of whether the bead is on the fan stops. If the fan is not controlled by the control temperature, the fan is only controlled by the light, that is, the open light fan starts, and the fan stops after 60 seconds after the light is closed |
|  |  | FAN0 fan without temperature control and light control (constant) |
| 12 | CH-A | (CH-A) Channel Mode Switching default value (CH-A) 1 Total FM 2 strobe 3 function 4 speed 5R 6G 7B 8W |
|  |  | (CH-b) Channel mode switching 1 Total adjustment 2R 3G 4B 5W 6 strobe 7 function 8 speed |
| 13 | F-14 | Dimmer refresh frequency (02k- -22k) hz default value (14khz) Note: The light takes effect from the new power, too low frequency may lead to flickering shooting but better dimming, too high frequency may lead to bad dimming but no flickering image, so the frequency is set according to the scene |
| 14 | L-01 | Dimmer curve (1-4) species, default value (01) 01 Linear dimming.02 Square dimming.03 Inverse-square dimming. The 04 S-shaped dimming is used. |
| 15 | 16bt | 16bt 16-bit dimming mode (soft dimming without blinking, and slower dimming speed) Default (16bt) |
|  |  | -8bt 8 bit dimming mode (dimming flashes slightly while dimming speed synchronizes with the console value change speed) |
| 16 | t001 | The t001 (000- -220) dimming speed is valid only for those used when selecting the 16bt dimming mode, which is not the menu default value for selecting the-8bt dimming mode (t001) |

4.DMX512 Channel Description (8-Channel)

| order number | function | explain |
| --- | --- | --- |
| Channel 1 | Total dimming | Total dimming, linear dimming, from dark to bright |
| Channel 2 | R aiming | R dimming, Linear dimming, from dark to bright |
| Channel 3 | G aiming | G dimming, linear dimming, from dark to bright |
| Channel 4 | B aiming | B dimming, linear dimming, from dark to bright |
| Channel 5 | W aiming | W dimming, linear dimming, from dark to bright |
| Channel 6 | Total frequency flash | Total strobe, from slow to fast |
| Channel 7 | FS | 0-5 Empty; 6-100 Jump change 101-150 Gradients 151-200 Vein change 201-255 Voice Control |
| Channel 8 | Functional speed | Functional speed, from slow to fast |

5.DMX512 Channel Description (4 Channel)

| order number | function | explain |
| --- | --- | --- |
| Channel 1 | R aiming | R dimming, Linear dimming, from dark to bright |
| Channel 2 | G aiming | G dimming, linear dimming, from dark to bright |
| Channel 3 | B aiming | B dimming, linear dimming, from dark to bright |
| Channel 4 | W aiming | W dimming, linear dimming, from dark to bright |

6.RDM description:

| order number | function | explain |
| --- | --- | --- |
| 1 | Feedback (Address code) | Free to change |
| 2 | Feedback (Channel mode) | Free to change |
| 3 | Feedback (equipment temperature) | Can't change |

7. Functional features:

The operation is very simple, humanized design, dimming without flicker, jitter phenomenon, etc. Can adapt to photography, photography, television station and other strict requirements on the lighting occasions.