Instructions for using the monochrome four-eye audience lights

- Graphic section:

A. function key

B. additional key

C. Dekey

D. Confirm key

A    B    C   D

Operation description: Press function key A to cycle different functions, and press B or C to modify its parameter values.Press the key D to confirm.

- Four-digital key board operation instructions

| order number | menu | Menu function | explain |
| --- | --- | --- | --- |
| 1 | C H1/4/9 | Channel mode selection 1CH/4CH/9CH | Press up and down to select the channel mode, and press OK to save. |
| 2 | d001 | DMX512 address code settings 001 | Press up and down to adjust the address code, and press OK to save. |
| 3 | E 255 | First-eye brightness adjustment 255 | Press the upper and lower keys to adjust the eye brightness, and press the OK keys to save. |
| 4 | F 255 | Second second eye brightness adjustment 255 | Press the upper and lower keys to adjust the eye brightness, and press the OK keys to save. |
| 5 | H 255 | Third, vision brightness adjustment 255 | Press the upper and lower keys to adjust the eye brightness, and press the OK keys to save. |
| 6 | L 255 | Fourth, vision brightness adjustment 255 | Press the upper and lower keys to adjust the eye brightness, and press the OK keys to save. |
| 7 | P r01 | Built-in program setting 01 | Press up and down to select the built-in program, and press OK to save. |
| 8 | S P01 | Built-in program speed settings 16 | Press up and down to select the built-in program speed, and press OK to save. |
| 9 | AUTO | From the walk | Press OK to enter, and press Save again. |
| 10 | AUDI | sound control | Press OK to enter, and press Save again. |
| 11 | DISP | Displays the inverted selection | Press up and down to select the forward and inverted displays. |

- DMX512 channel description

1 Channel mode

| order number | function | explain |
| --- | --- | --- |
| Channel 1 | Total dimming | (0-255) Total dimming, E, F, H, and L, linear dimming, from dark to light |

4 Channel mode

| order number | function | explain |
| --- | --- | --- |
| Channel 1 | E aiming | E dimming, (0-255) Linear dimming, from dark to bright |
| Channel 2 | F aiming | F dimming, (0-255) Linear dimming, from dark to bright |
| Channel 3 | H aiming | H dimming, (0-255) Linear dimming, from dark to bright |
| Channel 4 | L aiming | L dimming, (0-255) Linear dimming, from dark to bright |

9 Channel mode

| order number | function | explain |
| --- | --- | --- |
| Channel 1 | Total dimming | (0-255) Total dimming, E, F, H, and L, linear dimming, from dark to light |
| Channel 2 | Total frequency flash | (0-255) E, F, H, L total flash, from slow to fast |
| Channel 3 | E aiming | E dimming, (0-255) Linear dimming, from dark to bright |
| Channel 4 | F aiming | F dimming, (0-255) Linear dimming, from dark to bright |
| Channel 5 | H aiming | H dimming, (0-255) Linear dimming, from dark to bright |
| Channel 6 | L aiming | L dimming, (0-255) Linear dimming, from dark to bright |
| Channel 7 | Choose a bright eye | A combination of bright eyes. |
| Channel 8 | macro | (0-10) Enabling of the front 7 channels; (11-120) jump; (121-180): gradient; (181-244): jump, gradient; (245-255) sound control |
| Channel 9 | Macro function speed | 0-255 Up up, 0 fastest. |

In the address code state, after the multi-machine connection, if there is no reception console, the main and slave online function can be realized to operate one of the devices, and the rest in the address d001 state can synchronously run the program with the host.If the console signal is received after connecting the console, the address code d will become A, and the device in the address state can be controlled by the console.