Welcome to Laser 2.9 Software

I. The LCD display panel is shown in the figure below:

DMX-512 Mode

Auto    Mode

LED indicator

SelectMenu/Menu selection

1. Description of LED indicator status.

Laser Key RG: Slow laser lock status red and green slow flash

SafetyProtected R: ON G: OFF Closed light protection status red

DMX And Slave Mode RG: Fast DMX, Red And Green Flash In Slave Mode

Auto Mode R: OFF G: Slow Auto Mode Green Slow Flash

Music Mode, R: Fast G: ON voice mode, red flash + green

ILDA Mode R: OFF G: Fast ILda Interface Mode Green Flash

2. SelectMenu/Menu selection.

Select the required function menu by rotating left and right. When the menu is high, it is selected. Press OK/Return to select the function.

- LCD Chinese/English menu functions are shown in the following table:

| Level 1 menu | Secondary menu | Third-level menu |
| --- | --- | --- |
| - DMX-512 Mode DMX-512 model | DMX-512 Mode/DM X-512 Mode Address/Address code 001 Channel/Channel Expert/Professional | Address Setup/Address Code Setup 001 |
|  |  | Channel Setup Normal Mode/Normal Mode Expert Mode |
| - Auto Mode Self-propelled mode | Auto Mode/Self-propelled Mode Effect/Self-walking Effect Mix/All AutoSpeed/Self-propelled speed 50% | Auto Effect/Auto effect Mix Show/All Effects Lines Show/Line Effect Beams Show/Dotting Effect |
|  |  | Auto Speed/Self-propelled speed 50% |
| - Music Mode Voice activated mode | Music Mode/Voice Mode Effect Voice Effects Mix/All Music Speed/Voice Speed 50% | Music Effect/Voice Effect Mix Show/All Effects Lines Show/Line Effect Beams Show/Dotting Effect |
|  |  | Music Speed/Voice Speed 50% |
| - Laser Light Laser light settings | Laser Light/Laser light settings Scanner/Scan Speed 20 KPPS Color/Color Settings RGB Laser/Laser Analog Invert/Invert Settings Normal Image Size/Size Settings X Y Position/Position setting X Y | Scan Rate/Scan Speed 20KPPS |
|  |  | Color Setup/Color Setup Single/Single color RGY RBP GBC RGB WYPC |
|  |  | Laser Type/Laser settings TTL/TTL modulation Analog/Analog Modulation |
|  |  | Normal/Normal XY Invert X/invert X Invert Y/invert Y Invert XY/invert XY |
|  |  | Image Size/Size Settings Size X/Size X Size Y/Size Y |
|  |  | Position Setup Position X/position X Position Y/position Y |
| - Settings General settings | Settings/General Settings Reset Default/Reset Default SafetyProtected/SafetyProtected closed light Master Setup/Host Setup Back Light/Backlight Settings Language/Language settings Display Setup About System | Reset Default/Reset Default Yes/Yes No/No |
|  |  | SafetyProtected/SafetyProtected closed light Turn ON Turn OFF/Close |
|  |  | Master Setup/Host Setup Turn ON Turn OFF/Close |
|  |  | Back Light/Backlight Settings Normally ON Delay 15S OFF/delay 15s light closing |
|  |  | Language/Language settings Chinese/Chinese English/English |
|  |  | Display Setup Rotate 180deg/Rotate 180 degree NormalDisplay/Normal |
|  |  | Laser System2.9/Laser System2.9 Software/Software version Version 1.0/Ver1.0 YM:20190301001 |

- DMX-512 channel description:

1. Normal Mode is as shown in Table 10CH:

| Passageway |  | Channel value | Control content |
| --- | --- | --- | --- |
| CH1 | Total dimming | 0～255 | Total dimming 0 ~ 100% |
| CH2 | Red Light Brightness | 0～255 | Red light brightness 0-100% |
| CH3 | Green light brightness | 0～255 | Green light brightness 0 ~ 100% |
| CH4 | Blue light brightness | 0～255 | Blue light brightness 0 ~ 100% |
| CH5 | Pattern selection | 0～255 | Select a pattern for every 3 values |
| CH6 | Horizontal position | 1～255 | Horizontal position selection |
| CH7 | Vertical position | 1～255 | Vertical position selection |
| CH8 | Pattern size | 0～255 | Pattern selection from large to small |
| CH9 | Line scanning speed | 0～127 | Line scanning speed selection |
|  | Spot scanning speed | 128～255 | Selection of spot scanning speed |
| CH10 | Strobe | 1～127 | Strobe speed selection |
|  | Gradual drawing | 128～255 | Gradient position selection |
| CH11 | Pattern/effect library | 0～127 | Line effect library selection |
|  |  | 128～255 | Point effect library selection |
| CH12 | Mode selection | 0～26 | Manual control mode |
|  |  | 27～127 | Built-in self-propelled mode speed selection |
|  |  | 128～255 | Built-in voice mode speed selection |

- Expert Mode/Professional Mode 62CH Table:

1. Mode channel function.

| Passageway |  | Channel value | Control content |
| --- | --- | --- | --- |
| CH61 | Pattern/effect library | 0～127 | Line effect library selection |
|  |  | 128～255 | Point effect library selection |
| CH62 | Mode selection | 0～26 | Manual control mode |
|  |  | 27～127 | Built-in self-propelled mode speed selection |
|  |  | 128～255 | Built-in voice mode speed selection |

II. The channel functions of the six heads are as follows: Note (the channel

functions of each head are the same: 60CH = 10CH * 6).

| Passageway |  | Channel value | Control content |
| --- | --- | --- | --- |
| CH1 | Total dimming | 0～255 | Total dimming 0 ~ 100% |
| CH2 | Red Light Brightness | 0～255 | Red light brightness 0-100% |
| CH3 | Green light brightness | 0～255 | Green light brightness 0 ~ 100% |
| CH4 | Blue light brightness | 0～255 | Blue light brightness 0 ~ 100% |
| CH5 | Pattern selection | 0～255 | Select a pattern for every 3 values |
| CH6 | Horizontal position | 1～255 | Horizontal position selection |
| CH7 | Vertical position | 1～255 | Vertical position selection |
| CH8 | Pattern size | 0～255 | Pattern selection from large to small |
| CH9 | Line scanning speed | 0～127 | Line scanning speed selection |
|  | Spot scanning speed | 128～255 | Selection of spot scanning speed |
| CH10 | Strobe | 1～127 | Strobe speed selection |
|  | Gradual drawing | 128～255 | Gradient position selection |