# 60W BEAM MOVING HEAD

1. DMX512 Address Set (DMX Address setting)

This option is used to set the DMX address, press "Enter" button to Enter the Settings, press the "UP" key to address increasing number, against the "DOWN" key to address number, choose the right address after press "Enter" save the exit.

2. Run Mode (setting of lamp body working Mode)

This option is used to set the lamp body working mode, press "Enter" key to Enter the Settings, press the "UP" or "DOWN" button can work mode selection, work mode, in turn, have "DMX512", "missile", "Fast", "Sound" and "manual", choose the right press "Enter" to save out after working mode.

DMX512: console operation mode.

Slow: Slow down mode

Fast: Fast walking mode

Sound voice control mode

Manual: manual mode can be used for scene setting and can be online

3. Invert PAN/TILT

This option is to reverse set of X/Y direction, press "Enter" key to Enter the Settings, press the "UP" or "DOWN" key to X/Y axis is the reverse choice, choose "Yes" said X/Y reverse, select "NO" said X/Y axis does not reverse, choose the right direction after press "Enter" save the exit.

4. Display Set (screen setting)

This option is for liquid crystal display board backward, Chinese and English font display and backlight set, press "Enter" key to Enter the Settings, LCD backlight press "UP" or "DOWN" key to switch to choose, select "Yes" said LCD backlight has been bright long, isn't broken, select "NO" said there was NO button switch off after operation for 20 seconds, until there is reopened when the keystrokes. Select the Settings and press Enter to save the exit.

Invert Word fonts backwards

English/Chinese: Chinese and English translation

LCD Backlight on LCD Backlight setting

5. Manual Set (Manual mode setting)

This option is for the lamp body scene parameters setting, press "Enter" button to Enter to save, can be Run online, if the operation Mode of the lamp body (the Run Mode) to set up as the manul, manual Mode is effective, power outages, a boot before the next boot recoverable.

PAN                   X position

PAN_C                X axis unpole control

The PANMICO         X axis is fine-tuned

TILT                  Y axis position

TILT_C                Y axis has no polar control

TILTMICO             Y axis fine tuning

XY_SPEED            XY speed

DIM light              tuned

STROBE              stroboscopic

DIM_ R                 red light

DIM_ G                 green light

DIM _B                 blue light

DIM_ W                white dimming

STA_ CH              static effects

MOV_CH            dynamic effect

SHOW_SP            effect speed

MACRO_CH          macro function control

6. Sound Sence

This option is to switch lamp body acoustic sensitivity Settings, press "Enter" key to Enter the Settings, press the "UP" or "DOWN" key to the sensitivity of 0 ~ 100 degrees for choice, choose good sensitivity, press "Enter" to save after exit.

7. System Set (System parameter setting)

System Information (System Information) :

The version: V1 software version number

PAN Reset: X axis Reset status display Normal (Normal) Wrong

TILT Reset: Y axis Reset status display Normal (Normal) Wrong

Default Settings: restore factory Settings

Code Settings: password channel

When the password channel is used, the user needs to transfer the password to view and set the XY fine tuning and the lamp bead white balance parameter

The number of password is 8, press "UP" or "DOWN" key input, the factory password is "DOWN UP UP.

UP UP UP UP"

| Mode/ Channel | Value | Function |
| --- | --- | --- |
| 1 | 0-255 | PAN Movement 0-540° |
| 2 | 0-55 | PAN Movement stop |
|  | 56-155 | Forwards Pan rotation from fast to slow |
|  | 156-255 | Backwards Pan rotation from slow to fast |
| 3 | 0-255 | Pan Fine 16bit |
| 4 | 0-255 | TILT Movement 0-360° |
| 5 | 0-55 | TILT Movement stop |
|  | 56-155 | Forwards Pan rotation from fast to slow |
|  | 156-255 | Backwards Pan rotation from slow to fast |
| 6 | 0-255 | Tilt Fine 16bit |
| 7 | 0-255 | Speed Pan/Tilt movement |
| 8 | 0-255 | Dimmer intensity Intensity 0 to 100% |
| 9 | 0-55 | No function |
|  | 56-155 | Speed of the same pace is slow to fast（1HZ-25HZ) |
|  | 156-205 | Random stroboscopic |
|  | 206-255 | Lightning flashes |
| 10 | 0-255 | Red(0-Black, 255-100% Red) |
| 11 | 0-255 | Green (0-Black, 255-100% Green) |
| 12 | 0-255 | Blue (0-Black, 255-100% Blue) |
| 13 | 0-255 | White (0-Black, 255-100% White) |
| 14 | 0-24 | static effect 1 |
|  | 25-49 | static effect 2 |
|  | 50-74 | static effect 3 |
|  | 75-99 | static effect 4 |
|  | 100-124 | static effect 5 |
|  | 125-149 | static effect 6 |
|  | 150-174 | static effect 7 |
|  | 175-199 | static effect 8 |
|  | 200-224 | static effect 9 |
|  | 225-249 | static effect 10 |
|  | 250-255 | Effect flow display, speed is controlled by 16 channels |
| 15 | 0-49 | Dynamic effect 1, speed is controlled by 16 channels |
|  | 50-99 | Dynamic effect 2, speed is controlled by 16 channels |
|  | 100-149 | Dynamic effect 3, speed is controlled by 16 channels |
|  | 150-255 | Dynamic effect 4, speed is controlled by 16 channels |
| 16 | 0-255 | From fast to slow |
| 17 | 0-49 | No function |
|  | 50-99 | Static display |
|  | 100-149 | Dynamic display |
|  | 150-199 | Auto |
|  | 200-255 | sound |
| 18 | 0-250 | No function |
|  | 251-255 | reset |