# LED matrix (8+8) laser moving head light

Menu function:

ADDR                A001---A512 address code

SLND                MAST, SLAV

SHND                SOUD, AUTO Sound control, self-propelled selection

MANU               LASER1---LASER8,STROBE，Manual test

SENS                 Sound control sensitivity (0 is off, 100 is the most sensitive)

LED                  ON, OFF Backlight switch

TEST                 automatic testing

VER                  V100 Software version number

Product parameter introduction：

Input power: AC90V-240V

Power frequency: 50/60Hz

Power: 70W

Cooling method: air cooling

Maximum operating environment temperature: 45℃

Number of LEDs: 8

Single LED power: 3W

The average life of LED: 50000 hours

LED color temperature: 3200K

Number of lasers: 8

Laser model: 500MW/638NM red

Strobe: 1-25 times/sec or random

Number of DMX control channels: 30CH

DMX connector: 3Pin XLR

Operation mode: DMX/automatic/sound control

Vertical scan: 270°

Display mode: LED digital tube

30CH DMX512 ：

| CN | Features | DMX | description |
| --- | --- | --- | --- |
| 1 | Y axis | 0-255 | 0---270° |
| 2 | Y-axis fine adjustment | 0-255 | 0--1.0° |
| 3 | LED total dimming | 0-255 | Dimming 0%-dimming 100% |
| 4 | LED strobe | 0-255 | Strobe from slow to fast |
| 5 | LED1 | 0-255 | LED1 dimming from dark to bright |
| 6 | LED2 | 0-255 | LED2 dimming from dark to bright |
| 7 | LED3 | 0-255 | LED3 dimming from dark to bright |
| 8 | LED4 | 0-255 | LED4 dimming from dark to bright |
| 9 | LED5 | 0-255 | LED5 dimming from dark to bright |
| 10 | LED6 | 0-255 | LED6 dimming from dark to bright |
| 11 | LED7 | 0-255 | LED7 dimming from dark to bright |
| 12 | LED8 | 0-255 | LED8 dimming from dark to bright |
| 13 | Laser total dimming | 0-255 | Dimming 0%-dimming 100% |
| 14 | Laser strobe | 0-255 | Strobe from slow to fast |
| 15 | Laser 1 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 16 | Laser 2 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 17 | Laser 3 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 18 | Laser 4 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 19 | Laser 5 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 20 | Laser 6 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 21 | Laser 7 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 22 | Laser 8 | 0-127 | No function |
|  |  | 128-255 | Laser bright |
| 23 | LED macro function | 0-255 | Built-in LED macro function effect |
| 24 | LED macro function effect speed regulation | 0-255 | Built-in LED macro function effect speed regulation |
| 25 | Laser macro function effect | 0-255 | Built-in laser macro function effect |
| 26 | Laser macro function effect speed adjustment | 0-255 | Built-in laser macro function effect speed adjustment |
| 27 | LED+laser macro function effect | 0-255 | Built-in LED+laser macro function effect |
| 28 | LED+laser macro function effect speed adjustment | 0-255 | Built-in LED+laser macro function effect speed regulation |
| 29 | Y axis speed control | 0-255 | From fast to slow |
| 30 | Reset | 0-250 | No function |
|  |  | 251-255 | The lamp body is reset, valid for 3s. |