Four in one all cutting lamp

Please read the instructions carefully before use.
1.DMX Channel table
DMX Channel table

	35CH
	Function
	Channel value
	Effect

	1
	level
	000-255
	0-100%

	2
	pan fine 
	000-255
	0-100%

	3
	vertical
	000-255
	0-100%

	4
	Tilt Fine 
	000-255
	0-100%

	5
	Horizontal and vertical velocity
	000-255
	Horizontal and vertical speed, fast to slow

	6
	stroboscopic
	000-009
	consecration

	
	
	010-199
	Synchronize strobe slow to fast

	
	
	200-249
	Slow to fast asynchronous flicker

	
	
	250-255
	consecration

	7
	dimming
	000-255
	Dimming from 0% to 100%

	8
	cyan
	000-255
	From shallow to deep

	9
	magenta
	000-255
	From shallow to deep

	10
	yellow
	000-255
	From shallow to deep

	11
	color temperature
	000-255
	From shallow to deep

	12
	Color palette 1

	000-009
	white light

	
	
	010-019
	Color 1

	
	
	020-029
	Color 2

	
	
	030-039
	Color 3

	
	
	040-049
	Color 4

	
	
	050-059
	Color 5

	
	
	060-069
	Color 5+Color 4

	
	
	070-079
	Color 4+Color 3

	
	
	080-089
	Color 3+Color 2

	
	
	090-099
	Color 2+Color 1

	
	
	100-109
	Color 1+White Light

	
	
	110-255
	Slow to fast forward rotation

	13
	CRI (Color Rendering Index)
	000-127
	white light

	
	
	128-255
	Finger insertion

	14
	Fixed pattern
	000-009
	white light

	
	
	010-019
	Pattern 1

	
	
	020-029
	Pattern 2

	
	
	030-039
	Pattern 3

	
	
	040-049
	Pattern 4

	
	
	050-059
	Pattern 5

	
	
	060-069
	Pattern 6

	
	
	070-079
	Pattern 1: Shake from slow to fast

	
	
	080-089
	Pattern 2: Shake from slow to fast

	
	
	090-099
	Pattern 3: Shake from Slow to Fast

	
	
	100-109
	Pattern 4: Shake from Slow to Fast

	
	
	110-119
	Pattern 5: Shake from slow to fast

	
	
	120-129
	Pattern 6: Shake from slow to fast

	
	
	130-191
	Forward rotation fast to slow

	
	
	192-255
	Reverse slowly to fast

	15
	Rotating pattern
	000-009
	white light

	
	
	010-019
	Pattern 1

	
	
	020-029
	Pattern 2

	
	
	030-039
	Pattern 3

	
	
	040-049
	Pattern 4

	
	
	050-059
	Pattern 5

	
	
	060-069
	Pattern 6

	
	
	070-079
	Pattern 7

	
	
	080-089
	Pattern 1: Shake from slow to fast

	
	
	090-099
	Pattern 2: Shake from slow to fast

	
	
	100-109
	Pattern 3: Shake from Slow to Fast

	
	
	110-119
	Pattern 4: Shake from Slow to Fast

	
	
	120-129
	Pattern 5: Shake from slow to fast

	
	
	130-139
	Pattern 6: Shake from slow to fast

	
	
	140-149
	Pattern 7 shakes from slow to fast

	
	
	150-190
	Forward rotation fast to slow

	
	
	191-192
	stop

	
	
	193-255
	Reverse slowly to fast

	16
	Gobo Rotation
	000-127
	Pattern angle adjustment

	
	
	128-190
	Pattern turning fast to slow

	
	
	191-192
	stop

	
	
	193-255
	Pattern reversal slow to fast

	17
	Effect pattern
	000-009
	white light

	
	
	010-127
	Linear pattern

	
	
	128-255
	Pattern jitter is slow to fast

	18
	zoom
	000-255
	0-100%

	19
	focusing
	000-255
	0-100%

	20
	Focus fine-tuning
	000-255
	0-100%

	21
	prism
	000-127
	Prism free

	
	
	128-255
	Prism cutting in

	22
	Prism Rotation
	000-127
	Prism angle adjustment

	
	
	128-187
	Prism rotates slowly to quickly

	
	
	188-195
	Prism rotation stops

	
	
	196-255
	Prism reversal slow to fast

	23
	atomization
	000-003
	No atomization

	
	
	004-255
	Linear atomization

	24
	Cut on 1
	000-255
	0-100%

	25
	Cut on 2
	000-255
	0-100%

	26
	Cut off 1
	000-255
	0-100%

	27
	Cut down 2
	000-255
	0-100%

	28
	Cut left 1
	000-255
	0-100%

	29
	Cut left 2
	000-255
	0-100%

	30
	Cut right 1
	000-255
	0-100%

	31
	Cut right 2
	000-255
	0-100%

	32
	 Cutting Macro

	000-009
	non-functional

	
	
	010-247
	Graphic selection

	
	
	248-255
	Automatic graphic switching

	33
	Cutting rotation
	000-255
	0-100%

	34
	aperture
	0 - 127
	Linear aperture

	
	
	128 - 159
	Aperture fast in and slow out

	
	
	160 - 191
	Aperture slow in and fast out

	
	
	191 - 255
	Aperture fast in, fast out

	35
	reset
	0 - 25
	No effect

	
	
	26 - 76
	Small motor reset

	
	
	77 - 127
	XY motor reset

	
	
	128 - 255
	full reset


[image: image1.png]LED display- | Up key

DMIX signal-] — OK key

lamp I— Right key

Error signal |

lamp. — Left key
|~ Down key

The functions of the left and right buttons are the same. Return to the previous interface

Up and down to select Edit

OK key (OK key) executes the function, starts editing, and exits editing





1.  Display board function operation
· MENU : Cycle through the top menu or return to the previous menu
· ENTER: Enter the next menu or confirm and save the selected parameters
· UP: Up the menu, switch the selected parameter or increase the value
· DOWN: Down the menu, switch the selected parameter or decrease the value
Menu structure

	Set up
	Function

	Address code
	1 - 512
	Set address code

	passageway
	35CH
	Set channel

	Operating mode
	DMX512/go one's own way/Voice control/Manual
	Set operation mode

	Address,code Auto
	Open/shut
	The address code is automatically calculated. For example, the current address code is 1 and the channel mode is 16CH. After the address code is turned on automatically, press the set address code to automatically add 16, that is, the address code becomes 17

	Signal hold
	open/shut
	After opening, the original action will be maintained if there is no signal

	XYexchange
	open/shut
	X and Y channel exchange

	X-axis reversal
	open/shut
	X-axis reversal

	Y-axis reversal
	open/shut
	Y-axis reversal

	Encoder error correction
	open/shut
	Encoder error correction

	Hall error correction
	open/shut
	Hall error correction

	Restore settings
	Yes / no
	Restore factory settings (but do not restore trim data)


	Channel 
	Manual
	Function

	01
	level
	0-255
	

	02
	pan fine
	0-255
	

	03
	vertical
	0-255
	

	04
	Tilt Fine
	0-255
	

	05
	Horizontal and vertical velocity
	0-255
	

	06
	stroboscopic
	0-255
	

	07
	dimming
	0-255
	

	08
	cyan
	0-255
	

	09
	magenta
	0-255
	

	10
	yellow
	0-255
	

	11
	color temperature
	0-255
	

	12
	Color palette 1
	0-255
	

	13
	CRI (Color Rendering Index)
	0-255
	

	14
	Fixed pattern
	0-255
	

	15
	Rotating pattern
	0-255
	

	16
	Gobo Rotation
	0-255
	

	17
	Effect pattern
	0-255
	

	18
	zoom
	0-255
	

	19
	focusing
	0-255
	

	20
	Focus fine-tuning
	0-255
	

	21
	prism
	0-255
	

	22
	Prism Rotation
	0-255
	

	23
	atomization
	0-255
	

	24
	Cut on 1
	0-255
	

	25
	Cut on 2
	0-255
	

	26
	Cut off 1
	0-255
	

	27
	Cut down 2
	0-255
	

	28
	Cut left 1
	0-255
	

	29
	Cut left 2
	0-255
	

	30
	Cut right 1
	0-255
	

	31
	Cut right 2
	0-255
	

	32
	Cutting Macro
	0-255
	

	33
	Cutting rotation
	0-255
	

	34
	aperture
	0-255
	

	35
	reset
	0-255
	


	Calibration
	Effect

	Please enter a 4-digit password to use this function


	Display
	Effect

	Screen saver time
	15 seconds /30 seconds /60 seconds / off
	When the time arrives, return to the desktop

	backlight time
	15 seconds /30 seconds /60 seconds / off
	The screen will be off when the time reaches

	Screen reversal
	On / off
	Screen forward and reverse control

	language
	Chinese / English
	Select Chinese or English


	Reset
	Effect

	reset all
	Yes / no
	Reset all motors

	XY reset
	Yes / no
	XY motor reset

	Head reduction
	Yes / no
	Head small motor reset


	Bulb setting
	ffect

	Power on bright bubble
	Yes / no
	Automatic bubble lighting after power on reset

	Manual light bubble
	Yes / no
	Manual bubbling

	Half power
	Yes / no
	Whether half power mode is turned on


	DMX data monitoring
	Effect

	Monitor DMX512 data value of each channel


	Reset information
	Effect

	If the system reset is normal, the system is normal. If there is a reset error, the corresponding reset error message will be prompted


	Sensor information
	Effect

	X encoder
	*****
	Current encoder count value

	Y encoder
	*****
	Current encoder count value

	X encoder status
	00/11
	Encoder a and B signal line status

	Y encoder status
	00/11
	Encoder a and B signal line status

	X Hall
	0/1
	Hall state, hall is 1 at magnet position, otherwise it is 0

	Y Hall
	0/1
	Hall state, hall is 1 at magnet position, otherwise it is 0

	Color disc Hall
	0/1
	Hall state, hall is 1 at magnet position, otherwise it is 0

	Pattern disc Hall
	0/1
	Hall state, hall is 1 at magnet position, otherwise it is 0

	Focusing Hall
	0/1
	Hall state, hall is 1 at magnet position, otherwise it is 0


	Usage record
	Effect

	Use times, use time, bright bubble time, etc


	System version
	Effect

	System hardware version, software version number


Explain：

1. 1. when restoring factory settings, only the underlined parameters in the menu will be restored, and other parameters will not be automatically restored or reset.
2. 【Send local parameters】 menu, you can send the parameter settings of underlined items in the machine to another lamp or multiple lamps through DMX data line to avoid repeated setting work. A reset will be generated when the received lamps receive the correct parameters.
2. Troubleshooting
	
	Resolvent

	The lamp is not powered on and has no response at all
	· Check the fuse for damage
·  check whether the AC voltage is normal
· Switching power supply inside the lamp body may be damaged
·  the internal control board of the lamp body may be damaged

	Uncontrolled lamps
	·  check whether the DMX signal is normal. When it is normal, the DMX signal indicator on the display board will flash
·  check whether the address code is set correctly
·  check whether the channel mode is set correctly
· Check whether the DMX signal line is too long, or whether there are too many lamps connected in series

	Abnormal movement of XY axis
	· The optocoupler sensor may be damaged or improperly installed, so that the original correct position can also be corrected. Set it to no in the menu [XY error correction enable] option. Or adjust the optocoupler sensor circuit board to the appropriate position, or replace the circuit board



