Technical:

Voltage：AC90-240V 50/60HZ

Color: RGBW 4IN1

Four color Channel: 10CH

Operation: DMX512, Auto, Sound, Master-slave

LED Stage Lighting Operating Manual

LED mini spotlight

Please read manual carefully before use.

Thank you so much to choose this product; it is adopting the most advanced lamps, which is called LED lamps. It conquers a lot of disadvantages such as power wasting, short using life, high temperature, etc. It has advantages of power saving, long life (it can reach 60,000.00 hours in general circs), high brightness, fresh colors…It is the best stage illuminating and various effect decorative products.

WARNINGS:

1. Inspect the device carefully before installation in case of loose during transportation.

2. Don’t put the product on the shakable or moist place.

3 . Please keep the voltage stable, otherwise it will destroy the parts easily.

4. Check the voltage before turning on lamp. While power off, it needs 15minutes to cool before reconnecting the power.

5. Please keep the lamp dry and cool, .avoid working at 40℃ or above to guarantee its lifespan.

- LED display window function comparison table, (all the functions are selected and then press the D key to confirm)

| NO | Display | Function description |
| --- | --- | --- |
| 1 | D001 | Address code, (001-512) B and C keys add or subtract address code values |
| 2 | CC00 | Self-tossing variation |
| 3 | CP00 | gradual change |
| 4 | DE00 | Presentation mode |
| 5 | BEBE | Voice-activated mode |
| 6 | H000 | Motor travel setting |
| 7 | HP01 | Motor self-speed setting |
| 8 | R000 | Red brightness Settings |
| 9 | G000 | Green brightness setting |
| 10 | B000 | Blue brightness Settings |
| 11 | W000 | White brightness setting |
| 12 | REST | reset |

- DMX512channel description：Four  color Channel（10CH）

| Channel 1 | dimming | Total dimming |
| --- | --- | --- |
| Channel 2 | R | Red dimming |
| Channel 3 | G | Green dimming |
| Channel 4 | B | Blue dimming |
| Channel 5 | W | White dimming |
| Channel 6 | stroboflash | stroboflash |
| Channel 7 | Self-drive | The lamp moves by itself |
| Channel 8 | Self-driving speed | From slow to fast |
| Channel 9 | Zoom | Motor running |
| Channel 10 | Zoom speed | 0-250 motor speed |
|  | Reset | 251-255 Reset |

Features:

Color temperature: multicolour. Very simple operation, humanized design, dimming without flicker, jitter, etc. It can be adapted to occasions with strict lighting requirements such as photography, photography, and TV stations.