- LED OUTDOOR PIXEL BAR

- 9X10W 4in1 LED

- USER MANUAL

Thank you very much for choosing our product. For safety purpose, please read this manual carefully before your operation. This manual included installation and using information. Please install and operate it according to this manual.

- Part 1 Product

  - Product introduction

This product is designed for outdoor or indoor use. Suitable applications include wash or effect lighting for architectural, stage or nightclub applications. This product can also be installed for use in signage and advertising using the dynamic functions available with DMX512 control. Direct input of DMX512 signal allows the units to be controlled from any DMX512 controller. This product can be operated as a single unit or in multiple units for large applications.

1.2 Technical Specifications

Voltage: AC100-240V, 50/60Hz

- Power: 90W

Light Source: 9x10W (RGBW 4in1 LED)

Dimming: 0-100%

Lifespan: around 50,0000-100,000 hours

Control: DMX 512,9/14/36 channels

Control mode: DMX 512, Auto mode, Master/Slave mode

Beam angle: 25º

IP Rating: IP65

Net Weight：4.6Kg

Size: 515*91*177mm

1.3 Safety Warning

- IMPORTANT:

ALWAYS READ THE USER MANUAL BEFORE OPERATION. PLEASE CONFIRM THAT THE POWER SUPPLY STATED ON THE PRODUCT IS THE SAME AS THE MAINS POWER SUPPLY IN YOUR AREA.

● This product must be installed by a qualified professional.

● Always operate the equipment as described in the user manual.

● A minimum distance of 0.5m must be maintained between the equipment and combustible surface.

● The product must always be placed in a well ventilated area.

● Always make sure that the equipment is installed securely.

● DO NOT stand close to the equipment and stare directly into the LED light source.

● Always disconnect the power supply before attempting and maintenance.

● Always make sure that the supporting structure is solid and can support the combined weight of the products.

● The earth wire must always be connected to the ground.

● Do not touch the power cables if your hands are wet.

ATTENTION:

● This product left the place of manufacture in perfect condition. In order to maintain this condition and for safe operation, the user must always follow the instructions and safety warnings described in this user manual.

● Avoid shaking or strong impacts to any part of the equipment.

● Make sure that all parts of the equipment are kept clean and free of dust.

● Always make sure that the power connections are connected correct and secure.

● If there is any malfunction of the equipment, contact your distributor immediately.

● When transferring the product, it is advisable to use the original packaging in which the product left the factory.

● Shields, lenses or ultraviolet screens shall be changed if they have become damaged to such an extent that their effectiveness is impaired.

● The lamp (LED) shall be changed if it has become damaged or thermally deformed.

- PART 2 INSTALLATION

- 2.1 Mounting:

2.1-1 Hanging

The product can be mounted in a hanging position using the supporting bracket. The bracket should be secured to the mounting truss or structure using a standard mounting clamp. Please note that when hanging the unit a safety cable should also be used

.

2.1-2 Upright

The product can be mounted in an upright or sitting position using the supporting brackets.

- NOTE:

The product can be mounted at any angle and in any position. It is possible to further adjust the angle of the product using the two adjustment knobs located on the side of the fixture.

- PART 3 Display Panel Operation

MENU： Table1 function loops add

UP：    function value add

DOWN： function value decrease

ENTER： save the setting

3.2 Display panel operation instruction

| No. | Display | Function Instruction |
| --- | --- | --- |
| 1 | A001 | DMX address（001—512） |
| 2 | 14ch/9ch/36ch | Select Channel mode (14CH/ 9CH/ 36CH) |
| 3 | CC00 | 7 Colors Jump change,UP ro DOWN to change the speed |
| 4 | Ed00 | 7 Colors gradual change, pulse change（01—99）, UP or DOWN to change the speed |
| 5 | Er00 | 7 Colors Running1(01—99), UP or DOWN to change the speed. (Set ID, it can realize many fixtures running. |
| 6 | Po00 | 7 Colors Running 2(01—99), UP or DOWN to change the speed. (Set ID, it can realize many fixtures running. |
| 7 | tr00 | Running effect(00-08) UP or DOWN change efect. |
| 8 | tU00 | Running Speed (00-99) UP or DOWN change running speed |
| 9 | BE50 | 7 Colors Sound effect, UP or DOWN to change the sound sensitivity. |
| 10 | R200 | Red (000—255) From dark to bright |
| 11 | G200 | Green (0-255) From dark to bright. |
| 12 | B200 | Blue (0-255) From dark to bright. |
| 13 | W200 | White (0-255) From dark to bright. |
| 14 | ID00 | Fixture ID(01-28),UP or DOWN to change the ID value (use for running effect, it can realize many fixtures running. |

3.3 DMX Channel instruction

9-Channel Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| 1 | Dimmer | RGBW linear dimmer, from dark to bright |
| 2 | Strobe | RGBW Strobe, from slow to fast |
| 3 | Function selection 1 | 0--25:No function 26--35:No function 36--45：7 Colors running (jump change), (can set different ID to realize many fixtures running, controlled by channel 1-5) 46--55: 7 Colors running (gradual change), (can set different ID to realize many fixtures running, controlled by channel 1-5 ) 56--65: Color selection, controlled by channel 1) 66--75: Jump change (controlled by channel 1) 76--85:Gradual change, pulse change (controlled by channel 1) 86--95: Gradual change effect 1 (controlled by channel 1-5) 96--105： Gradual change effect 2 (controlled by channel 1-5) ...... 236--245: Running effect 15 (controlled by channel 1-5) 246--255: Sound control (controlled by channel 1-5) |
| 4 | Function selection 2 | 0-5: No function 6--255：250 kinds of Running effect selection （controlled by channel 1-6) |
|  | Function 3 | 7 Colors running direction, (0-127 running left, 128-255 running right |
| 5 | Function speed | Speed from slow to fast |
| 6 | Red dimming | Red linear dimming, from dark to bright |
| 7 | Green dimming | Green linear dimming, from dark to bright |
| 8 | Blue dimming | Blue linear dimming, from dark to bright |
| 9 | White dimming | White linear dimming, from dark to bright |

- 14-Channel Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| 1 | Dimmer | RGBW linear dimmer, from dark to bright |
| 2 | Strobe | RGBW Strobe, from slow to fast |
| 3 | Function selection 1 | 0--25:No function 26--35:No function 36--45：7 Colors running (jump change), (can set different ID to realize many fixtures running, controlled by channel 1-5) 46--55: 7 Colors running (gradual change), (can set different ID to realize many fixtures running, controlled by channel 1-5 ) 56--65: Color selection, controlled by channel 1) 66--75: Jump change (controlled by channel 1) 76--85:Gradual change, pulse cange (controlled by channel 1) 86--95: Gradual change effect 1 (controlled by channel 1-5) 96--105： Gradual change effect 2 (controlled by channel 1-5) ...... 236--245: Running effect 15 (controlled by channel 1-5) 246--255: Sound control (controlled by channel 1-5) |
| 4 | Function selection 2 | 0-5: No function 6--255：250 kinds of Running effect selection （controlled by channel 1-6) |
|  | Function 3 | 7 Colors running direction, (0-127 running left, 128-255 running right |
| 5 | Function speed | Speed from slow to fast |
| 6 | LED 1 dimming | RGBW dimming |
| 7 | LED 2 dimming | RGBW dimming |
| 8 | LED 3 dimming | RGBW dimming |
| 9 | LED 4 dimming | RGBW dimming |
| ...... | ....... | .......... |
| 14 | LED 9 dimming | RGBW dimming |

- 36-Channel Mode

| Channel | Funciton | Instruction |
| --- | --- | --- |
| 1 | R1 dimming | R1 linear dimming, from dark to bright |
| 2 | G1 dimming | G1 linear dimming, from dark to bright |
| 3 | B1 dimming | B1 linear dimming, from dark to bright |
| 4 | W1dimming | W1 linear dimming, from dark to bright |
| 5 | R2 dimming | R2 linear dimming, from dark to bright |
| 6 | G2 dimming | G2 linear dimming, from dark to bright |
| 7 | B2 dimming | B2 linear dimming, from dark to bright |
| 8 | W2dimming | W2 linear dimming, from dark to bright |
| ...... | ....... | ........ |
| 36 | W9 dimming | W18linear dimming, from dark to bright |

- Part 4 Maintenance

The cleaning of internal must be carried out periodically to optimize light output. Cleaning frequency depends on the environment in which the fixture operates: damp, smoky or particularly dirty surrounding can cause greater accumulation of dirt on the fixture’s optics.

● Clean with soft cloth using normal glass cleaning fluid.

● Always dry the parts carefully.

● Clean the external optics at least every 20 days.

● Clean the internal optics at least every 30/60 days – Up to vary products.

- Please ask professional technical to clean waterproof products and make sure the fixture will be reassembled well for waterproof.