
MegaPointe IP66 580W LED Beam Spot Wash Moving Head Light

[image: image37.jpg]



User Manual

Note: This manual include important information on how to install and use it safely. Please read it carefully before install and operate as required. At the same time, please keep this manual properly for emergencies.

Contents

1. Safety warning   …………………………………………………………3

2. User Instructions ……………………………………………… …………6
3. Lamp appearance …………………………………………………………8
4. Installation……………………………………………………………………9
5. Function setting………………………………………………………………13 
6. Menu operation ………………………………………………………………14
7. DMX channel …………………………………………………………………16
8. Technical parameters ………………………………………………………30

9. Photo metric date ……………………………………………………………35
Accessories:

The following random accessories have been packaged with light, please check and verify:

	Name
	QTY
	Remark

	Signal cable 
	1
	

	Power cable
	1
	

	User manual 
	1
	

	Safety rope
	1
	charge

	G clamp
	2
	charge

	Omega clamp
	2
	charge


When this product leaves the factory, the performance is intact and the packaging is complete. All users should strictly abide by the warnings and operating instructions stated in this manual. Any damage caused by misuse is not covered by the company's warranty. Failures and problems caused by ignoring the operating manual are not within the scope of the dealer's responsibility.

This manual is subject to technical changes without notice.

Note: The company's products are calculated based on the company's delivery date. The warranty period of the whole machine is one year, and the light source is not within warranty.

1   SAFETY INSTRUCTIONS
	[image: image1.png]



Notice

Please read the safety requirements in this section carefully before installing, energizing, operating or repairing !


The following important safety signs will be used in this product manual

	[image: image2.png]



	[image: image3.png]



	[image: image4.png]



	[image: image5.png]



	[image: image6.png]



	[image: image7.png]



	[image: image8.png]





	[image: image21.jpg]



	● After receiving the lamp, please unpack and check whether there is any damage caused by transportation. If there is damage caused by transportation, please do not use this lamp, and contact the dealer or manufacturer as soon as possible.

● The manufacturer will not be held responsible for the losses caused by not following this manual and changing the lamps at will.

● Please note that the damage to the lamp caused by the modification of the lamp is not covered by the warranty.

● If you have any questions or suggestions, please contact the dealer or manufacturer in time.



	[image: image22.jpg]EER




	● When the lamp is in use, it must be kept away from high temperature, fire, electric shock, vibration and strong light environment.

● Qualified operators can carry out installation, maintenance and repair, and ensure that they operate in strict accordance with the operating procedures of this manual

● There are no children who can be repaired by the user inside this lamp, please do not open the lamp to repair by yourself.

	[image: image23.jpg]BB RRIEET

Paas





	● Do not look directly at the light source of the lamp (especially for patients with epilepsy) to avoid damage to the eyes!

● Do not connect this lamp to any other dimming device.

● If visible damage occurs to the light source, lens or display protective cover on the lamp, it is damaged to the extent of failure. If cracks or deformation occur, please stop using it and contact the manufacturer to replace the original accessories, otherwise normal use will be affected.

● The installation position of the luminaire should be such that it will not be stared at at a distance of less than 4 meters for a long time.

	[image: image24.jpg]=
7
BARFRA

BARN R




	●  Before starting to operate the lamp, please check whether all the covers (or shells) have been installed and whether the screws have been tightened reliably. It is forbidden to use the lamp with the cover (or shell) open.

● Keep the lamp clean and do not touch the lamp glass directly with your hands.

● During operation, it is recommended to wear necessary protective equipment, such as goggles, gloves, etc.

	[image: image25.jpg]i

LARKEFE




	● The person who connects the electrical part must have the corresponding qualifications before the operation.

● Before installation, please make sure that the power supply voltage you are using matches the voltage marked on the lamp.

● Each lamp should be properly grounded, and electrical installation should be carried out in accordance with relevant standards.

● Do not use power cords with damaged insulation, and do not overlap power cords with other wires.

● When the lamp is not in use or cleaning, please hold the power cord plug to unplug it, do not pull or directly pull the power cord.

● Make sure that the power cord used complies with safety regulations.

● If the lamp is not waterproof or dustproof, please be careful not to short-circuit the lamp when it is exposed to rain or moisture during the operation of the lamp.

● Do not turn on or off the lamp repeatedly within a short period of time, as this will shorten the life of the lamp light source or other electrical components.

	[image: image26.jpg]Ry




	● There is a safety rope connection hole at the bottom of the lamp. For safety reasons, please pass the safety rope equipped with the lamp through the safety rope connection hole for auxiliary lifting.

● Before any installation, maintenance and cleaning of the lamp, please make sure that the power supply has been cut off!

	[image: image27.jpg]



	● Under normal working conditions at room temperature, the temperature of the outer metal surface of the lamp (including the surface of the radiator) will reach 50°C at the highest temperature.

● There will be smoke and peculiar smell before the light is turned on for the first time. This is a normal process and does not mean that the lamp is defective.

● When using, do not touch the shell with bare hands to avoid burns!



	[image: image28.jpg]/5

Sk R





	● Please do not install the lamp directly on the surface of ordinary combustible materials.

● Do not project the lamp directly on combustible objects, and the distance between the lamp and the illuminated object is more than 10 meters.

● The lamp should be installed in a well-ventilated place, and the distance from the wall should be kept at least 50 cm. At the same time, please check whether the fan and ventilation holes are unblocked.

●Please do not expose the front glass to sunlight or other strong light sources from any angle. The lens may focus the light in the lamp, which may cause a potential fire hazard.

	[image: image9.png]



	● This product complies with the "General Technical Requirements Standards for the Recycling of Waste Electrical and Electronic Products"

[image: image29.png]


● When the product reaches the scrap standard and needs to be scrapped, the customer can scrap and recycle the product.


 2   USER INSTRUCTIONS

Cleaning and Maintenance

When the lamp cannot be started, please check whether the power fuse of the lamp is blown. The lamp is equipped with an overheating protection device. The protection device will automatically cut off the power when it is overheated. When this happens, please check whether the fan is running normally and whether the fan and fan net are blocked by dust. Find out the fault and repair it before starting the lamp.

Because substances such as smoke oil and dust tend to adhere to the lens and reduce the light output, in order to ensure the reliable use of the lamp, it is very necessary to keep the lamp clean. The cooling fan should be cleaned every 15 days. The cleaning of internal and external optical lenses, mirrors, and coated color filters must be performed periodically to optimize light output.

    The cleaning frequency depends on the operating frequency of the lamp and the surrounding environment. When cleaning, use a soft cloth and general glass cleaning products to clean. It is recommended to clean the external optical system at least once every 20 days, and clean the internal optical system at least once every 30/60 days. Keep the lens clean and do not directly touch the optics of the lamp with your hands.

●[image: image30.png]


Before performing any maintenance and cleaning the lamp, please make sure that the power has been cut off!

●Only qualified technicians can carry out maintenance work. 

●Before maintenance,the power must be completely cut off!

[image: image31.png]


●In order to prevent sunlight or other light from directly irradiating the head lens and focusing on the inside of the lamp body to cause high temperature damage to the lamp, before the lamp is powered off, please use the Y-axis control channel to keep the lamp head down.

● Do not wipe the lamp housing with organic solvents such as alcohol to avoid damage.

● Do not use any solvents containing chemical components to clean the coated color filters.

● Lubrication

In order to maintain the smooth rotation of the gobo and the smooth movement of the focus lens, it is recommended to give the focus lens every two months

The 4 guide rails and the 4 guide rails of the magnifying lens are lubricated once. Good and high-temperature-resistant lubricating oil/grease should be used.

●Ordinary Troubleshooting
	Fault description
	Take countermeasures

	The lamp can't work normally
	check if the power fuse is blown

Check if the bulb is in good condition

	The lamps are not controlled by the controller
	Check if the DMX start address of the lamp is set correctly

Check if the XLR signal cable is damaged

	The lamps work intermittently
	Check whether the fan is working normally, and whether dust is blocking the fan and the fan net

	The light is dim and the brightness is obviously reduced
	Check whether the LED light source has reached the expiration date

Check whether the internal and external optical system is clean

	The beam is impure (has a halo)
	Clean the dust and oil from the lens and other parts

	Serious beam distortion
	Check if the lens is broken

Clean the lens dust or oil


 3   LAMP APPEARANCE 

[image: image32.png]


[image: image33.png]


[image: image34.jpg]


[image: image35.jpg]


[image: image36.jpg]


[image: image10.png]



 4   INSTALLATION 

  [image: image11.jpg]


 [image: image12.png]



Take out the 2 hooks and 1 safety rope that came with the machine, use the 4 hook quick knobs attached to the hooks to install the 2 hooks on the bottom of the luminaire, and then hang the luminaire on the fixed bracket with the hooks and twist Tighten the hook and lock bolts to fix the whole lamp. Please confirm that the lamp is installed firmly and reliably, and ensure that the hoisting position has enough strength to bear the weight of the lamp. For safety reasons, please follow the warning shown in the figure above to pass the safety rope provided with the lamp through the safety hole at the bottom of the lamp holder for auxiliary lifting to ensure safety.

Warning!!
● The hook is only used when hoisting the lamp. It is strictly forbidden to use the hook as a handle to carry the lamp. Please use the handle when carrying.

● For safety reasons, please use a safety rope that can withstand 10 times the weight of the lamp to pass through the safety hole of the lamp for auxiliary lifting.

● Power connection:

The power cord connection is as follows:

L (fire wire) = brown wire

E (ground wire) = yellow/green two-color wire

N (center line) = blue line

When connecting the power supply, please note that the power supply voltage and frequency must be consistent with the voltage and frequency marked on the nameplate of the lamp. When multiple lamps are used at the same time, it is recommended to connect the power supply of each lamp separately, so that each lamp can be individually controlled on/off.

● When connecting the power supply, the ground wire (yellow/green two-color wire) must be safely grounded, and the electrical installation must comply with all relevant standards.

● If you have any questions about electrical installation, please do not operate and consult a qualified electrician.

● Connection of control signal:

[image: image13.png]DMX_OUT

DMX_QUT

s

3

DMX51e
PIN FUNCTION
1 GND

e DATA-
3 DATA+
4 NO FUNCTION
S NO FUNCTION





The connection between the lamp and the controller and between the lamp and the lamp must use a two-core shielded wire with a diameter of not less than 0.5 mm. Please use a 3-pin (included) XLR plug/seat to connect the DMX512 output/inlet of the lamp. The connection between the XLR plug/socket and the wire is shown in the list above. It must be noted that the 3-core of the XLR plug/seat cannot be in contact with the inner shell and the core and the core cannot be in contact during the connection. Except for the connection methods shown in the list above, the XLR plug/seat and the XLR control line cannot be connected in any other way. This product receives the international standard DMX512 (1990) control signal.

Use the XLR-XLR control cable to connect the DMX output port of the controller to the DMX input port of the first slave, and connect the DMX output port of the first slave to the DMX input port of the second slave, and so on By analogy, until all the slaves are connected, and finally the circuit is connected to the signal output port of the last lamp to complete the controller mode connection. As shown below:

          
[image: image14.png]



DMX loop plug

In the "controller mode", the DMX output port of the last lamp must be connected to the DMX loop plug. This circuit is inserted between pin 2 and pin 3 of the "Canon" plug and connected with a resistance of about 120Ω (OHM) (as shown in the figure below). Plug this loop into the output port of the last lamp, which can effectively avoid the noise and reflection caused by the DMX512 signal during the transmission process.

[image: image15.emf]PIN 3


PIN 2


120¦¸


3


1


2


DMX


回路插联接


在


XLR


插头的


2


针和3针之间联接�


一个


120    (OHM)


的电阻，并将其


插在最后一台灯具的


DMX 


输出插座上。




PIN 3

PIN 2

120Ω

3

1

2

DMX

回路插联接

在

XLR

插头的2针和3针之间联接

一个

120    (OHM)

的电阻，并将其

插在最后一台灯具的

DMX 

输出插座上。


 5   FUNCTION SETTING
●Display operation

[image: image16.emf]
To view or modify the function settings of the lamp, press any key (in the power-on state) to light up the screen, and then press the UP and DOWN keys to enter the corresponding menu of the lamp. There are corresponding sub-menus in the function operation main menu, and each menu represents a specific function of the lamp. For details, see the sixth point "Operation Menu" below.

1. In the lamp function menu setting page, press the UP and DOWN keys to select the corresponding function.

2. When operating the menu, the MENU button is the menu button, and the ENTER button is the confirmation button. Press the ENTER button to save your changes or enter the submenu. Press the UP or DOWN button to modify the value (increase or decrease the value).

Press the MENU button to return to the previous menu. If you don't press it, the system will automatically return to the display state.

● DMX address setting

When using the controller to control multiple lamps, each lamp must set the DMX start address to receive the signal from the controller and respond correctly. This product has two control modes, namely standard mode and simplified mode. Take the standard mode as an example: the product has a total of 34 channels, then the DMX start address of the first lamp is set to 001, the second is 035, the third is 069 and so on.
The menu setting of this fixture optimizes the address setting. The operations for several setting address codes are as follows:

1 Select "Previous" or "Next", the lamp will automatically calculate the address code of the next or previous set according to the current address code and channel data, which can be set quickly;

2 Click the address code value to enter the value editing window, where any valid address code can be set, the fixture will automatically obtain the current channel number of the fixture, and automatically filter the unusable address code (512-current channel number).

3 The lamps support the RDM protocol, and the address code of the lamps can be set remotely through RDM.


 6   MENU OPERATIONS

	Option
	Instruction

	DMX set
	0-512
	

	
	up
	

	
	down
	

	setting
	DMX channel
	34CH/39CH

	
	RDM function
	On/off

	
	Language 
	Chinese/english

	
	Screen flip
	Stay/delete

	
	Dmx signal
	On/off

	
	Screen protect
	On/off

	
	X Invert
	On/off

	
	Y Invert
	On/off

	
	X Y Swap
	On/off

	
	X Y Encoder
	On/off

	
	Restore Defaults
	Yes/no

	Running mode
	Auto mode
	Dmx,sound auto

	
	Manual control
	01x.....(dmx running)

	
	Lamp reset
	

	
	XY reset
	

	
	MT reset
	

	System info.
	System version
	

	
	Temperature info
	

	
	Fan info
	

	
	System time
	

	
	Sensor monitoring
	

	
	System error
	

	
	DMX monitor
	

	Dimmer curve
	Dimmer 
	Exponential

	
	
	Linear

	
	
	Sine

	
	
	Logarithmic

	Factory setting
	
	


 7   DMX CHANNEL 
34/39 channel mode sheet

	39CH


	34CH
	Function
	Channel Value
	    Description

	CH1
	CH1
	Pan
	0-255
	0-540°

	CH2
	CH2
	Pan fine
	0-255
	Pan fine

	CH3
	CH3
	Tilt
	0-255
	0-270°

	CH4
	CH4
	Tilt  Fine
	0-255
	Tilt fine

	CH5
	
	X, Y speed
	0
	Standard Mode

	
	
	
	1
	Fastest mode

	
	
	
	2-255
	Speed 0.2-25.5 second

	CH6
	CH6
	Macro function
	0-139
	No function 

	
	
	
	pause for 3 seconds in the DMX value and get following function

	
	
	
	30-34
	High CRI(CRI=90+)

	
	
	
	35-39
	Standard CRI(CRI=80)

	
	
	
	40-139
	No function

	
	
	
	140-149
	Scan reset

	
	
	
	150-159
	Color reset

	
	
	
	160-169
	Gobo wheel reset

	
	
	
	170-179
	Dimmer reset

	
	
	
	180-189
	Zoom,focus,prism,frost reset

	
	
	
	190-199
	Effect wheel reset

	
	
	
	200-209
	All reset

	
	
	
	210-229
	No function

	
	
	
	230-239
	Turn off lamp

	
	
	
	240-255
	No function

	CH7
	CH7
	C
	0-255
	White-C

	CH8
	CH8
	M
	0-255
	White-M

	CH9
	CH9
	Y
	0-255
	White-Y

	CH10
	CH10
	Color wheel
	Any position

	
	
	
	0
	white

	
	
	
	9
	Color 1 Dark Red

	
	
	
	18
	Color 2 Dark Blue

	
	
	
	27
	Color 3 Yellow

	
	
	
	37
	Color 4 Light green

	
	
	
	46
	Color 5 Magenta

	
	
	
	55
	Color 6 Mauve

	
	
	
	64
	Color 7 Pink

	
	
	
	73
	Color 8 Dark green

	
	
	
	82
	Color 9 CTO2700k

	
	
	
	91
	Color 10 Blue

	
	
	
	101
	Color 11 Orange

	
	
	
	110
	Color 12 CTO 3200k

	
	
	
	119
	Color 13 UV

	
	
	
	128-129
	White

	
	
	
	Color position

	
	
	
	130-134
	Color 1 Dark Red

	
	
	
	135-138
	Color 2 Dark Blue

	
	
	
	139-143
	Color 3 yellow

	
	
	
	144-147
	Color 4 Light Green

	
	
	
	148-152
	Color 5 Magenta

	
	
	
	153-157
	Color 6 Mauve

	
	
	
	158-161
	Color 7 Pink

	
	
	
	162-166
	Color 8 Dark Green

	
	
	
	167-171
	Color 9 CTO2700k

	
	
	
	172-176
	Color 10 Blue

	
	
	
	177-180
	Color 11 Orange

	
	
	
	181-185
	Color 12 CTO 3200k

	
	
	
	186-189
	Color 13 UV

	
	
	
	190-215
	Forward flow from fast to slow

	
	
	
	216-217
	stop

	
	
	
	218-243
	Reverse flow from slow to fast

	
	
	
	244-249
	backup

	
	
	
	250-255
	Random Color Fast-Slow

	CH11
	
	Color wheel fine
	0-255
	Color wheel fine

	CH12
	CH11
	Color macro
	0
	No function

	
	
	
	1-2
	Filter 4 (amber)

	
	
	
	3-4
	Filter 10 (medium yellow)

	
	
	
	5-6
	Filter 19 (light yellow)

	
	
	
	7-8
	Filter 26 (bright red)

	
	
	
	9-10
	Filter 58 (light purple)

	
	
	
	11-12
	Filter 68 (sky blue)

	
	
	
	13-14
	Filter 71 (royal blue)

	
	
	
	15-16
	Filter 79 (blue)

	
	
	
	17-18
	Filter 88 (light green)

	
	
	
	19-20
	Filter 90 (dark yellow-green)

	
	
	
	21-22
	Filter 100 (warm yellow)

	
	
	
	23-24
	Filter 101 (yellow)

	
	
	
	25-26
	Filter 102 (Light Amber)

	
	
	
	27-28
	Filter 103 (Straw)

	
	
	
	29-30
	Filter 104 (Dark Amber)

	
	
	
	31-32
	Filter 105 (Orange)

	
	
	
	33-34
	Filter 106 (Light Red)

	
	
	
	35-36
	Filter 111 (Dark Pink)

	
	
	
	37-38
	Filter 115 (Peacock Blue)

	
	
	
	39-40
	Filter 116 (Turquoise)

	
	
	
	41-42
	Filter 117 (Steel Blue)

	
	
	
	43-44
	Filter 118 (Light Blue)

	
	
	
	45-46
	Filter 119 (Dark Cyan)

	
	
	
	47-48
	Filter 120 (Dark Blue)

	
	
	
	49-50
	Filter 121 (Green filter)

	
	
	
	51-52
	Filter 128 (Pink)

	
	
	
	53-54
	Filter 131 (Sky blue)

	
	
	
	55-56
	Filter 132 (Medium blue)

	
	
	
	57-58
	Filter 134 (Golden amber)

	
	
	
	59-60
	Filter 135 (Dark golden amber)

	
	
	
	61-62
	Filter 136 (Light purple)

	
	
	
	63-64
	Filter 137 (Special purple)

	
	
	
	65-66
	Filter 138 (Light green)

	
	
	
	67-68
	Filter 139 (Dark green)

	
	
	
	69-70
	Filter 141 (Bright blue)

	
	
	
	71-72
	Filter 147 (Apricot blue)

	
	
	
	73-74
	Filter 148 (Bright rose red)

	
	
	
	75-76
	Filter 152 (Light gold)

	
	
	
	77-78
	Filter 154 (Light rose red)

	
	
	
	79-80
	Filter 157 (Pink)

	
	
	
	81-82
	Filter 158 (Dark orange)

	
	
	
	83-84
	Filter 162 (Light Amber)

	
	
	
	85-86
	Filter 164 (Flame Red)

	
	
	
	87-88
	Filter 165 (Daylight Blue)

	
	
	
	89-90
	Filter 169 (Lilac)

	
	
	
	91-92
	Filter 170 (Dark Purple)

	
	
	
	93-94
	Filter 172 (Lake Blue)

	
	
	
	95-96
	Filter 179 (Chrome Orange)

	
	
	
	97-98
	Filter 180 (Dark Purple)

	
	
	
	99-100
	Filter 181 (Congo Blue)

	
	
	
	101-102
	Filter 197 (Weird Blue)

	
	
	
	103-104
	Filter 201 (C.T. Blue)

	
	
	
	105-106
	Filter 202 (Half C.T. Blue)

	
	
	
	107-108
	Filter 203 (Quarter C.T. Blue)

	
	
	
	109-110
	Filter 204 (C.T. Orange)

	
	
	
	111-112
	Filter 205 (Half C.T. Orange)

	
	
	
	113-114
	Filter 206 (Quarter C.T. Orange)

	
	
	
	115-116
	Filter 247 (Filter Green)

	
	
	
	117-118
	Filter 248 (half filter green)

	
	
	
	119-120
	Filter 281 (three quarters C.T. blue)

	
	
	
	121-122
	Filter 285 (three quarters C.T. orange)

	
	
	
	123-124
	Filter 352 (glacier blue)

	
	
	
	125-126
	Filter 353 (light blue)

	
	
	
	127-128
	Filter 715 (UV color)

	
	
	
	129-130
	Filter 778 (light orange)

	
	
	
	131-132
	Filter 793 (light rose red)

	
	
	
	133-255
	spare

	CH13
	CH12
	Speed
	0-255
	CMY speed from big to small

	CH14
	CH13
	Color speed
	0
	No function

	
	
	
	1-255
	CMY,color wheel time(0.1-25.5s)

	CH15
	CH14
	Zoom,focus,frost,prism speed
	0
	No function

	
	
	
	1-50
	Prism(0.1-5s)

	
	
	
	51-100
	frost(0.1-10s)

	
	
	
	101-255
	Zoom,focus(0.1-25.5s)

	CH16
	CH15
	Effect wheel
	0-19
	No function

	
	
	
	20-127
	Any position

	
	
	
	128-170
	From white to all (speed from fast to slow)

	
	
	
	171-213
	From white to half  (speed from fast to slow)

	
	
	
	214-255
	From half to all  (speed from fast to slow)

	CH17
	CH16
	Effect Wheel Rotate
	0
	No function

	
	
	
	1-127
	Forward flow from fast to slow

	
	
	
	128
	No function

	
	
	
	129-255
	Reverse flow from slow to fast

	CH18
	CH17
	Effect Macro
	0-3
	No function 

	
	
	
	set the zoom=16,     

projection distance=5 meters.

focus check value

And then get the following effect

	
	
	
	4-5
	Macro 1

	
	
	
	6-7
	Macro 2

	
	
	
	...
	...

	
	
	
	20-21
	Macro 9

	
	
	
	22-23
	Macro 10

	
	
	
	Use the follow effect, effect pattern channel and effect wheel channel automatic hide

	
	
	
	24-25
	Macro 1

	
	
	
	26-27
	Macro 2

	
	
	
	
	...

	
	
	
	40-41
	Macro 9

	
	
	
	42-43
	Macro 10

	
	
	
	44-255
	backup

	CH19
	CH18
	Fixed gobo wheel
	0-3
	white

	
	
	
	Position

	
	
	
	4-9
	Gobo1

	
	
	
	10-15
	Gobo2

	
	
	
	16-21
	Gobo3

	
	
	
	22-27
	Gobo4

	
	
	
	28-33
	Gobo5

	
	
	
	34-39
	Gobo6

	
	
	
	40-45
	Gobo7

	
	
	
	46-51
	Gobo8

	
	
	
	52-57
	Gobo9

	
	
	
	58-63
	Gobo10

	
	
	
	64-69
	Beam 1

	
	
	
	70-75
	Beam 2

	
	
	
	76-81
	Beam 3

	
	
	
	82-87
	Beam 4

	
	
	
	Gobo shake

	
	
	
	88-95
	Gobo1 shake from slow to fast

	
	
	
	96-103
	Gobo2 shake from slow to fast

	
	
	
	104-111
	Gobo3 shake from slow to fast

	
	
	
	112-119
	Gobo4 shake from slow to fast

	
	
	
	120-127
	Gobo5 shake from slow to fast

	
	
	
	128-135
	Gobo6 shake from slow to fast

	
	
	
	136-143
	Gobo7 shake from slow to fast

	
	
	
	144-151
	Gobo8 shake from slow to fast

	
	
	
	152-159
	Gobo9 shake from slow to fast

	
	
	
	160-167
	Gobo10 shake from slow to fast

	
	
	
	168-175
	Beam 1 shake from slow to fast

	
	
	
	176-183
	Beam 2 shake from slow to fast

	
	
	
	184-191
	Beam 3 shake from slow to fast

	
	
	
	192-199
	Beam 4 shake from slow to fast

	
	
	
	200-201
	White 

	
	
	
	202-222
	Forward flow from fast to slow

	
	
	
	223-243
	Reverse flow from slow to fast

	
	
	
	244-249
	Backup

	
	
	
	250-255
	Random gobo From fast to slow

	CH20

	CH19
	Rotation gobo wheel
	Gobo rotation setting channel 21/20

	
	
	
	0
	No function

	
	
	
	1-4
	white

	
	
	
	5-7
	Gobo1

	
	
	
	8-10
	Gobo2

	
	
	
	11-13
	Gobo3

	
	
	
	14-16
	Gobo4

	
	
	
	17-19
	Gobo5

	
	
	
	20-22
	Gobo6

	
	
	
	23-25
	Gobo7

	
	
	
	26-28
	Gobo8

	
	
	
	29-31
	Gobo9

	
	
	
	Gobo rotation setting channel 21/20

	
	
	
	32-34
	Gobo1

	
	
	
	35-37
	Gobo2

	
	
	
	...
	....

	
	
	
	50-52
	Gobo7

	
	
	
	53-55
	Gobo8

	
	
	
	56-59
	Gobo9

	
	
	
	Gobo shake from slow to fast, gobo rotation setting channel 21/20

	
	
	
	60-67
	Gobo1

	
	
	
	68-75
	Gobo2

	
	
	
	
	....

	
	
	
	116-123
	Gobo8

	
	
	
	124-129
	Gobo9

	
	
	
	Gobo shake from slow to fast, gobo rotation setting channel 21/20

	
	
	
	130-137
	Gobo1

	
	
	
	138-145
	Gobo2

	
	
	
	
	....

	
	
	
	186-193
	Gobo8

	
	
	
	194-199
	Gobo9

	
	
	
	200-201
	white

	
	
	
	202-222
	Forward flow from fast to slow

	
	
	
	223-243
	Reverse flow from slow to fast

	
	
	
	244-249
	Backup

	
	
	
	250-255
	Random gobo From fast to slow

	CH21
	CH20
	Gobo rotation
	Setting channel 20/19

	
	
	
	0-255
	Gobo rotation any position

	
	
	
	Setting channel 20/19

	
	
	
	0
	No function 

	
	
	
	1-127
	Forward flow from fast to slow

	
	
	
	128
	No function 

	
	
	
	129-255
	Reverse flow from slow to fast

	CH22
	
	Rotation gobo fine
	0-255
	Fine0-100%

	CH23
	CH21
	Prism 1
	0-3
	No function

	
	
	
	Setting channel 24/22

	
	
	
	4-7
	Prism1

	
	
	
	8-11
	Prism2

	
	
	
	12-15
	Prism3

	
	
	
	Setting channel 24/22

	
	
	
	16-19
	Prism1

	
	
	
	20-23
	Prism2

	
	
	
	24-27
	Prism3

	
	
	
	28-255
	Backup

	CH24
	CH22
	Prism 1 rotation
	Setting channel 23/21

	
	
	
	0-255
	Any position

	
	
	
	Setting channel 23/21

	
	
	
	0
	No function

	
	
	
	1-127
	Forward flow from fast to slow

	
	
	
	128
	Stop

	
	
	
	129-255
	Reverse flow from slow to fast

	CH25
	CH23
	Prism 2
	0-3
	No function

	
	
	
	Setting channel 26/24

	
	
	
	4-7
	Prism1

	
	
	
	8-11
	Prism2

	
	
	
	12-15
	Prism3

	
	
	
	Setting channel 26/24

	
	
	
	16-19
	Prism1

	
	
	
	20-23
	Prism2

	
	
	
	24-27
	Prism3

	
	
	
	28-255
	Backup

	CH26

	CH24
	Prism 2 rotation
	Setting channel 25/23

	
	
	
	0-255
	Any position

	
	
	
	Setting channel 25/23

	
	
	
	0
	No function

	
	
	
	1-127
	Forward flow from fast to slow

	
	
	
	128
	Stop

	
	
	
	129-255
	Reverse flow from slow to fast

	CH27
	CH25
	Prism macro
	0-3
	No function

	
	
	
	4-5
	Prism 1 effect

	
	
	
	...
	

	
	
	
	26-27
	Prism 12 effect

	
	
	
	Setting channel 28/26

	
	
	
	28-29
	Prism 1 effect

	
	
	
	...
	...

	
	
	
	50-51
	Prism 12 effect

	
	
	
	Setting channel 28/26

	
	
	
	52-53
	Prism 13 effect

	
	
	
	...
	...

	
	
	
	66-67
	Prism 20 effect

	
	
	
	68-255
	Backup

	CH28
	CH26
	Prism macro rotation
	Setting channel 27/25

	
	
	
	0-255
	Any position

	
	
	
	Setting channel 27/25

	
	
	
	0
	No function

	
	
	
	1-127
	Forward flow from fast to slow

	
	
	
	128
	No function

	
	
	
	129-255
	Reverse flow from slow to fast

	CH29
	CH27
	Beam effect
	Prism,Frost,static gobo,rotate gobo automatic shielding(expect light frost),get the following function

	
	
	
	0-3
	No function

	
	
	
	Setting channel 30/28

	
	
	
	4-7
	Beam 1

	
	
	
	8-11
	Beam 2

	
	
	
	12-15
	Beam 3

	
	
	
	16-19
	Beam 4

	
	
	
	Setting channel 30/28

	
	
	
	20-23
	Beam 1

	
	
	
	24-27
	Beam 2

	
	
	
	28-31
	Beam 3

	
	
	
	32-25
	Beam 4

	
	
	
	36-255
	Backup

	CH30
	CH28
	Beam rotation
	Setting channel 29/27

	
	
	
	0-255
	Any position

	
	
	
	Setting channel 29/27

	
	
	
	0
	No function

	
	
	
	1-127
	Forward flow from fast to slow

	
	
	
	128
	No function

	
	
	
	129-255
	Reverse flow from slow to fast

	CH31
	CH29
	Frost
	0
	No function

	
	
	
	Light atomization (0-100%), maximum time 10 seconds

	
	
	
	1-50
	Linear frost  Mild

	
	
	
	51-53
	All frost

	
	
	
	54-63
	Frost from slow to fast,slow off,fast open

	
	
	
	64-73
	Frost from fast to slow,slow on,fast off

	
	
	
	74-83
	Frost from fast to slow,slow off,slow on

	
	
	
	84-86
	No function

	
	
	
	Medium atomization (0-100%), maximum time 10 seconds (cannot be used simultaneously with Prism 1; Prism 1 has higher priority than this atomization)

	
	
	
	87-136
	Linear frost 0%-100%

	
	
	
	137-139
	Full frost

	
	
	
	140-149
	Frost from slow to fast, slow off, fast on

	
	
	
	150-159
	Frost from fast to slow, slow on, fast off

	
	
	
	160-169
	Frost from fast to slow, slow off, slow on

	
	
	
	170-172
	No function

	
	
	
	Medium atomization. Note: When stacking atomization, please avoid Prism 1 (the maximum duration of medium atomization 0-100% is 10 seconds).

	
	
	
	173-222
	Linear frost 0%-100%

	
	
	
	223-225
	Full frost

	
	
	
	226-235
	Frost from slow to fast, slow off, fast on

	
	
	
	236-245
	Frost from fast to slow, slow on, fast off

	
	
	
	246-255
	Frost from fast to slow, slow off, slow on

	CH32
	CH30
	Zoom
	0-255
	Zoom 0-100%

	CH33
	
	Zoom fine
	0-255
	Zoom fine

	CH34
	CH31
	Focus
	0-255
	Focus 0-100%

	CH35
	
	Focus fine
	0-255
	Focus fine

	CH36
	CH32
	spare
	0-255
	

	CH37
	CH33
	Strobe
	0-31
	Off 

	
	
	
	32-63
	On 

	
	
	
	64-95
	Synchronous strobe From slow to fast

	
	
	
	96-127
	open

	
	
	
	128-143
	Effect strobe from slow to fast

	
	
	
	144-159
	Effect strobe from fast to slow

	
	
	
	160-191
	open

	
	
	
	192-223
	Random strobe,from slow to fast

	
	
	
	224-255
	on

	CH38
	CH34
	dimmer
	0-255
	0-100%

	CH39
	
	Dimmer fine
	0-255
	Dimmer fine


 8   TECHNICAL PARAMETER
Electrical parameters:
Voltage: 100V~240V AC, 50/60Hz

Power: 580W @ 220V

Power factor: PF>0.9

Light source specification:

Light source:400W high brightness Module LED 

Color temperature:6500K-7000K

Life time:50000 hours

colour:

1 color wheel: 13 colors + white,half-color rainbow effect
Linear CMY function, color can be adjusted;

Can be mixed to make the color richer.

Gobo wheel:

1 rotating gobo wheel: 9 gobos + white

The pattern plate adopts a detachable design, which is more convenient to replace the pattern plate

1 fixed gobo wheel:10 gobos + white+4 beam

Dynamic Effects wheel: 1pcs with rotation function

Prism:

6pcs prism, dual-direction rotation
Zoom: 

DMX linear zoom adjustment

Focus:

DMX linear focus adjustment, super smooth focus adjustment.

Strobe/Dimming:

Electronic strobe, 0.3-25 times/second,linear dimming

Frost:

Independent frost lens, softer light

Rotation angle: 

Use three-phase stepping motor, fast, quiet, with automatic return function.

Horizontal:540°

vertical: 270° 

Beam Angle:

2-35°
Control mode:
DMX512 signal

39/34CH channel mode
Other functions:

Horizontal and vertical speed adjustable

Use powerful silent fan, LED temperature display, reduce power due to overheating, and ensure LED lifetime

Modular structure for easy maintenance

Color screen with Chinese and English menus.

Displays DMX values for each channel.

The screen automatically rotates according to the lighting fixture's hanging direction.

Features an automatic error reporting function.

Protection level:

Protection grade: IP66
weight: 

Net weight: 25.5kg 

Gross weight: 30.0kg

packing size: (mm)

520(L)*350(W)*860(H)

Working temperature:

Maximum ambient temperature 50°C


Fixed gobo wheel       Rotation Gobo Wheel          Effect Wheel

Color Wheel


CMY



Prism Wheel
Body size: (mm)
[image: image17.jpg]e p—

127

ﬂ-ﬂn g — =4

bae DSS\@




[image: image18.jpg]o :ﬂmjoo-zDBS &
1
b

476

01





 9   PHOTOMETRIC DATA
[image: image19.png]LUX:

PHOTOMETRICS DIAGRAM 2°
lux2° 360000 300000 174000 86000

Distance(m) 2m 3m 5m 8 m




[image: image20.png]LUX:

PHOTOMETRICS DIAGRAM35°
lux35° 9400 5000 2200 1600

Distance(m) 2m 3m 5m 8 m




arm





base





body





Warning!!


To ensure safety, please pass the safety rope through the connection hole to ensure safety.





Clamps





Safety cable





回路器





Signal in





DMX512





Signal out





Signal out





Lopper





dmx loop plug connection


Connect a 120Ω (OHM) resistor before pin 2 and pin 3 of the XLR plug, and plug it into the DMX output of the last lamp








3
3

