LED 90W BEAM LIGHT USER MANUAL

Specification：

Lighting source：90 LED white module

Max power consumption：250W

Voltage：wide voltage input switch power supply 100V-240V 50-60Hz

Color temperature：7000K-8000K

Lighting source lifespan：50000 hours

Spot angle：0-2°

Control mode：DMX512、master-slave

Channel mode:16CH/20CH

Support RDM function,DMX software upgrade function

Matched network：Artnet,sACN,network agreement

X/Y scan：horizontal 540°，vertical：270°

Color wheel：13 kinds of color+pure white，rainbow effect，Arbitrary positioning

Fixed gobo：12 kinds of fixed gobo，shake water flow effect

6colorful plate：one independent color 6 segments color plate,make gobo+rainbow effect.

Frost plate：frost flexible mirror

Dimming：0-100﹪ liner dimming

Strobe：sync pulse strobe,randomly pulse strobe

Prism :8 facets prism，two-way rotating effect

Focus：spot can arbitrary adjust to clear

Display：colorful liquid crystal display,Chinese and English bilingual,reversible display

High efficiency heat dissipation system,whole process temperature monitoring,overheat self-protection,non risk of lighting source damage.

Product dimension：31*23*49cm

Net weight:10.8kg

Gross weight：12kg

| LED 90W MENU TABLE |  |  |
| --- | --- | --- |
| MAIN | SECOND | THIRD |
| Dmx Address | 1 - 512 |  |
| Fixture | Dmx Mode | STD/EXT |
|  | Run Mode | DMX/Host/Sound |
|  | Pan Invert | Close/Open |
|  | Tilt Invert | Close/Open |
| Manual | Pan | 0 - 255 |
|  | Pan Fine | 0 - 255 |
|  | Tilt | 0 - 255 |
|  | Tilt Fine | 0 - 255 |
|  | ⁞ | ⁞ |
|  | Reset | 0 - 255 |
| Information | Time | Current Time：xxx H |
|  |  | Show Time：xxx H |
|  |  | Power Count：xxx |
|  | Sensor | Type Information |
|  | Temperature | Head Temp：xxx C |
|  |  | Fan Speed：xxxx RPM |
|  | Software Version | Panel：Vx.xxx |
|  |  | XY Motor：Vx.xxx |
|  |  | Motor：Vx.xxx |
| Factory | Password | 0 - 255 |
|  | Pan | 0 - 255 |
|  | Tilt | 0 - 255 |
|  | Dimmer | 0 - 255 |
|  | Color | 0 - 255 |
|  | FixGobo | 0 - 255 |
|  | Colorful | 0 - 255 |
|  | Frost | 0 - 255 |
|  | Prism1 | 0 - 255 |
|  | Prism2 | 0 - 255 |
|  | Zoom | 0 - 255 |
| Reset | All Reset | Cancel/Run |
|  | Factory | Cancel/Run |
| Display | Language | CH/EN |
|  | Display Flip | Close/Open |
|  | Display Mode | Show/60s |

| 90W LED CHANNEL TABLE |  |  |  |
| --- | --- | --- | --- |
| 20CH | 16CH | DMX values | Channel function |
| 1 | 1 | 0－255 | X |
| 2 | 2 | 0－255 | The X axis fine-tuning |
| 3 | 3 | 0－255 | Y |
| 4 | 4 | 0－255 | Y fine-tuning |
| 5 | * | 0－255 | X, y axis speed |
| 6 | 5 |  | STROBE |
|  |  | 0-3 | Shut down |
|  |  | 4-103 | Stroboscopic from slow to fast |
|  |  | 104-107 | medallion |
|  |  | 108-207 | Pulse stroboscopic from slow to fast |
|  |  | 208-212 | medallion |
|  |  | 213-225 | Random slow stroboscopic |
|  |  | 226-238 | Random medium speed stroboscopic |
|  |  | 239-251 | Random fast stroboscopic |
|  |  | 252-255 | medallion |
| 7 | 6 | 0-255 | Dimmer |
| 8 | * | 0-255 | The dimming fine-tuning |
| 9 | 7 |  | Color piece |
|  |  | 0-4 | The white light |
|  |  | 5-9 | White light plus color 1 |
|  |  | 10-14 | Color 1 |
|  |  | 15-19 | Color 1 plus color 2 |
|  |  | 20-24 | 2 colors |
|  |  | 25-29 | Color 2 plus color 3 |
|  |  | 30-34 | The color of 3 |
|  |  | 35-39 | Color 3 plus color 4 |
|  |  | 40-44 | 4 color |
|  |  | 45-49 | Color 4 plus color 5 |
|  |  | 50-54 | Color 5 |
|  |  | 55-59 | Color 5 plus color 6 |
|  |  | 60-64 | Color 6 |
|  |  | 65-69 | Color 6 plus color 7 |
|  |  | 70-74 | Seven colors |
|  |  | 75-79 | Color 7 plus color 8 |
|  |  | 80-84 | Color 8 |
|  |  | 85-89 | Color 8 plus color 9 |
|  |  | 90-94 | Color 9 |
|  |  | 95-99 | Color 9 plus color 10 |
|  |  | 100-104 | Color 10 |
|  |  | 105-109 | Color 10 plus color 11 |
|  |  | 110-114 | Color 11 |
|  |  | 115-119 | Color 11 plus color 12 |
|  |  | 120-124 | Color 12 |
|  |  | 125-129 | Color 12 plus color 13 |
|  |  | 130-134 | The color of 13 |
|  |  | 140-149 | The white light |
|  |  | 115－199 | From fast to slow water |
|  |  | 200-205 | Water stop |
|  |  | 206-255 | From slow to fast water |
| 10 | * | 0-255 | Color piece of fine-tuning |
| 11 | 8 |  | Fixed pattern |
|  |  | 0-9 | The white light |
|  |  | 10-19 | Pattern 1 |
|  |  | 20-29 | Pattern 2 |
|  |  | 30-39 | Pattern 3 |
|  |  | 40-49 | Pattern 4 |
|  |  | 50-59 | Pattern 5 |
|  |  | 60-69 | Pattern 6 |
|  |  | 70-79 | Pattern 7 |
|  |  | 80-89 | Pattern 8 |
|  |  | 90-99 | Pattern 9 |
|  |  | 100-109 | Pattern 10 |
|  |  | 110-119 | Pattern 11 |
|  |  | 120-127 | Pattern 1 jitter (from slow to fast) |
|  |  | 128-135 | Pattern 2 jitter (from slow to fast) |
|  |  | 136-143 | Pattern 3 jitter (from slow to fast) |
|  |  | 144-151 | Pattern 4 jitter (from slow to fast) |
|  |  | 152-159 | Pattern 5 jitter (from slow to fast) |
|  |  | 160-167 | Pattern 6 jitter (from slow to fast) |
|  |  | 168-175 | Pattern 7 jitter (from slow to fast) |
|  |  | 176-183 | Pattern 8 jitter (from slow to fast) |
|  |  | 184-191 | Pattern 9 jitter (from slow to fast) |
|  |  | 192-199 | Pattern 10jitter (from slow to fast) |
|  |  | 200-207 | Pattern 11jitter (from slow to fast) |
|  |  | 208-230 | Pattern flowing (fast to slow) |
|  |  | 231-232 | Water stop |
|  |  | 233-255 | Pattern backflow (from slow to fast) |
| 12 | 9 | 0－255 | focusing |
| 13 | * | 0－255 | Focusing fine-tuning |
| 14 | 10 |  | 7 colour |
|  |  | 0-127 | empty |
|  |  | 128-255 | 7 colour |
| 15 | 11 |  | atomization |
|  |  | 0-255 | Linear atomization |
| 16 | 12 |  | Prism |
|  |  | 0－127 | empty |
|  |  | 128-255 | Prism opens |
| 17 | 13 |  | Rotation of prism |
|  |  | 0－63 | Linear orientation of prism |
|  |  | 64－126 | Clockwise flow from fast to slow |
|  |  | 127－128 | Water stop |
|  |  | 129－191 | Counterclockwise flow from slow to fast |
|  |  | 192－255 | Positive and negative rotation (from slow to fast) |
| 18 | 14 |  | empty |
|  |  | 0－127 | empty |
|  |  | 128-255 | empty |
| 19 | 15 |  |  |
|  |  | 0－63 | empty |
|  |  | 64－126 | empty |
|  |  | 127－128 | empty |
|  |  | 129－191 | empty |
|  |  | 192－255 | empty |
| 20 | 16 |  | reset |
|  |  | 200－205 | Lamps and lanterns is reset |
|  |  |  | empty |