Head-shaking beam lamp

USER MANUAL

（TFT DISPLAY）

[image: image1.jpg]DMX512
5PIN

[y

+ SIGNAL

DMX512
3PIN

= ol
SIGNAL SCREEN3L;
SIGNAL +





Please read the manual carefully before using it.

CONTENTS
1

Chapter 1 Installation and attention

1

1.
Maintenance

1

2.
Statement

1

3.
Safety Precaution

1

4.
Cable connection（DMX）

2

5.
Rigging (Optional)

3

6.
RDM NOTE

4

Chapter 2  Panel operation

4

1.
Brief

2.
Operation
5
1.
Operate fixture with knob or button
5
4

2.
Parameter value setting

3.
Boolean parameter setting
6
4.
Sub Menu（Parameter）
6
5.
Anti-false touch operation of key
7
3.
Operation and parameter instruction
8
1.
DMX Address setting
8
2.
Fixture operating mode setting
9
3.
Set display
10
4.
Scene
10
5.
Set light run parameter
12
6.
Status and information
13
Chapter 3  Channel description
15
1.
Channel table
15


Chapter 1 Installation and attention 

Maintenance

To reduce the risk of electrical shock or fire, do not expose this unit to rain or moisture.
Intermittently using will extend this item’s service life.

Please clear the fan, fan net, and optical lens in order to keep good work state.
Do not use the alcohol or any other organic solvent to wipe the shell.
Statement

The product has perfect performance and integrity packing. All users should be strictly complying with the warning and operating instructions as stated. Or we aren’t in charge of any result by misusing. Any damage resulting by misuse is not within the Company’s warranty. Any fault or problem caused by neglecting the manual is also not in the charge of dealers.

   Note: All information is subject to change without prior notice.

Safety Precaution
In order to guarantee the product’s life, please don’t put it in the damp places or even the environment over 60degress.

Always mount this unit in safe and stable matter.

Install or dismantle should operate by professional engineer.
Using lamp，the change rate of power voltage should be within±10%，If the voltage is too high，it will shorten the light’s life; If it’s not enough, will influence the effect.
Please restart it 20 minutes later after turning off light , until full-cooling. Frequent switching will reduce the life span of lamps and bulbs; intermittent using will improve the life of bulbs and lamps.
In order to make sure the product is used well, please read the Manual carefully.
Cable connection（DMX）

Use a cable conforming to specifications EIA RS-485: 2-pole twisted, shielded, 120Ohm characteristic impedance, 22-24 AWG, low capacity. Do not use microphone cable or other cable with characteristics differing from those specified. The end connections must be made using XLR type 3 or 5-pin male/female connectors. A terminating plug must be inserted into the last projector with a resistance of 120Ohm (minimum 1/4 W) between terminals 2 and 3.Figure 1 shows a signal line connection diagram (the fixture in the figure is an example picture and does not represent the real appearance of this product).
IMPORTANT: The wires  must not make contact with each  other or with the metal casing of the connectors. The casing itself must be connected to the shield braid and to pin 1 of the connectors.

[image: image15.jpg]



Figure 1  Diagram of the DMX Cable connection

Rigging (Optional)

As shown in Figure 2 (the fixture in the figure is an example picture and does not represent the real appearance of this product),this equipment can be positioned and fixed by clamp in every direction of the stage. Locking system makes it easy to fasten to the bracket. 

Attention! Two clamps is needed to fix the equipment. Every clamp is locked by fastener of 1/4 kind. Fastener can only be locked clockwise.

Attention! Fasten a safety string to the additional hole of side aluminum piece. The secondary accessory can not hang on the delivery handle. Nip the equipment on bracket.

Check if rigging clamp (not including the one inside) damaged or not? If stand ten times weight as the equipment. Make sure the architecture can stand ten times weight as all the equipments, clamps, wirings and other additional fixtures.
Screws for clamping must be fixed firmly. Take one M12 screw (Grade 8.8 or higher) to clamp bracket, and then screw the nuts.
Level the two hanging points at the bottom of clamp. Insert fastener to the bottom, lock the two levers by 1/4 rotating clockwise; then install another clamp.
Install on safety string which stands at least ten times weight as equipment. Terminal of the accessory is designed for clamps.
Make sure pan/tilt lock unlocked or not. Keep the distance more than 1M from equipment to flammable material or lighting source.
[image: image2.png]


[image: image3.png]



Figure 2  Diagram of the Installation 
RDM Note

RDM is an extended version of DMX512-A protocol. It is a remote device management protocol. Traditional DMX512 protocol communication is one-way communication. The protocol is based on RS-485 bus. RS-485 is a time-sharing multi-point, half-duplex protocol. Only one port is allowed to output at the same time. So, when using RDM, we should pay attention to it. The following points:

To use console or host device that supports RDM host protocol.

Use bidirectional signal amplifier, traditional one-way signal amplifier is not suitable for RDM protocol, because the RMD protocol needs feedback data, the use of one-way amplifier will block the return of data, resulting in no search fixture;

All fixture must be set to DMX mode to ensure only one host on the cable.

A 120 ohm impedance matching resistor must be inserted between terminals 2 and 3 of the terminal plug. When the signal line is longer, reducing the signal reflection will make the differential signal more stable and beneficial to the quality of communication.

When the fixture appears to accept DMX control, but can not been search by RDM host, first check the signal amplifier, and then check whether the signal line 2, 3 lines have bad contact.
Chapter 2  Panel operation

Brief

The diagram of the display panel show as Figure 3, above area is title for fixture description, the white font in the lower right corner shows the fault status of the fixture (when the fault information is not viewed, it displays "ERR", otherwise it displays "NOR"), and the status bar below shows the signal of the current fixture , fixture status, communication status, etc. (the panel in the figure is an example picture and does not represent the real appearance of the product panel，please select the panel of the same type as your product for reference.).
RDM protocol is embed in fixture, user set DMX address via cable using the controller support RDM function. when fixture was search by controller, displayer will echo ‘RDM’ indicate this RDM is work. 

Note: Prevent damage the TFT displayer, Can not use sharp objects chick displayer.
[image: image4.png]16CHA[01:01:37 | NOR

| ADDR | | SCENE|
| MODE | 001 \ ADVA
DISP \ 01.01 \ STAT

001 ] K B





Figure 3-1 Diagram of the Five-buttons display panel
[image: image5.png]16 CHA| 01:01:37 | NOR

ADDR SCENE

wooe 001 [rowa|

DISP 01.01 STAT
001 [ ]

up

e





Figure 3-2  Diagram of the knob display panel
Operation

Operate fixture with touch knob or button

The left area is the display area, the right area is the input area, you can use the key or knob to control the cursor to select the item that needs to be set or viewed, and press the "ENTE" button to complete the operation.

For the knob shown in Figure 3-2, the cursor can be controlled up or down by rotating in different directions, and pressing the knob can confirm it. If you want to go back, turn the knob to move the cursor to the back button on the display, press the knob to confirm and return.

Parameter value setting

When the selected item is value need to been modified, the dialog shown in Figure 4 will popup.

       [image: image6.png]Adjust value quick

Morror — | 01 O





Figure 4  Dialog of value setting

Modify value：The desired value can be set by pressing the "Up" and "Down" buttons or by turning the knob.

Save Value：After setting the data by pressing the button, press the "ENTE" button, the values are immediately saved to the internal memory, and the saved values are applied to the fixture the next time the machine is turned on.

Boolean parameter setting

when the selected parameters is a Boolean value (such as ON or OFF), can directly modify setting by chick corresponding item, the setting will been saved right now. 
When the parameter is a key item, chick corresponding item, a dialog shown in Figure 5 will been popup ask for the confirm. Chick ‘sure’ to confirm.

[image: image7.png]SURE

NO




Figure 5  Dialog of confirm

Sub Menu（Parameter）
[image: image8.png](P

001 ==

16CHA 01.01 || Chan

Figure 6-1 Address setting

Scene Time 0.0s|
Control Mode OFF
01.Colour 000
02.Strobe 000
03.Dimmer 000

Figure 6-4 Scene Settings

ey —

Auto Run

Sound Ctrl

Scene Mode Auto
M/S Choose Auto
Light Switch OFF

!

English

A

pull

Screen Rot Auto
DMX Indicate Mode3
Screen Light 010

Figure 6-2 Run Settings

OFF I

Tilt Invert OFF
P/T Rectify ON
Pan Offset 010
Tilt Offset 010
Data hold OFF

Figure 6-3 Display Settings

Error Logging
Fixture Status
Version C8.1.0
Light time 0:00
Total time 15:32

Figure 6-5 Advanced setting

Figure 6-6 Status Settings




Figure 6  Diagram of the Parameter menu

Operation and parameter instruction

Chick item of main menu, enter corresponding sub menu shown in Figure 6, In main menu, chick 1/6 function button into corresponding parameter menu.

DMX Address setting
Enter page show in Figure6-1, can set fixture DMX address, channel mode and so on.

[image: image9.png]Prev

16CHA 01.01 || Chan





Figure 6-1  

The menu settings of fixture have optimized the setting of addresses. Several settings of the address are as follows:

Select " Prev " or "Next", the fixture will be based on the current address and channel mode, automatically calculate the next or last address, make address setting can quickly;

Click on the address value, you can enter the numeric editing window, where you can set any valid address, fixture system automatically get the current number of channels, automatically filter the unusable address (512 - the current number of channels).

Fixture support RDM protocol, remote address can be set through RDM.

Provide one buttons:

Channel mode:: you can choose different channel modes by cycle.
Fixture operating mode setting

[image: image10.png]Auto Run

Sound Ctrl
Scene Mode Auto
M/S Choose Auto

Light Switch OFF





Figure 6-2  
Through the page shown in Figure 6-2, the operating mode of the fixture can be set and the lamp can be controlled. The fixture supports four operating modes (DMX mode, auto mode, voice control mode and scene mode). Detailed parameter settings can be refer in the previous section. Specific parameter descriptions are as follows:

operating mode

	DMX Ctrl
	DMX mode, receive DMX signal, RDM signal

	Auto Run
	Fixture run automatically according to built-in programs

	Sound Ctrl
	When the fixture detects a strong sound, the fixture automatically runs a scene according to the built-in program, otherwise it will stay the last scene

	Scene Mode 01
	runs in a set scene, which supports most of the custom editing of 10 scenes.

	
	1~10
	outputs the specified scene

	
	Auto
	Automatically loops the output scene in the set scene time (non-zero) order, and the scene with time 0 automatically ignore

	M/S Choose
	Master and slave selection, non-DMX mode takes effect, select the mode of data output, fixture detect DMX cable state automatic switch output, prevent data conflicts

	
	Master
	fixture runs built-in program. If DMX has no signal, it outputs data (synchronization), otherwise it does not output data.

	
	Slave
	fixture runs built-in program and do not output data

	
	Auto
	If DMX has no signal, the fixture will runs built-in program. Otherwise, the fixture will run in DMX Mode(follow DMX).

	Lamp switch
	(Lamp light source) pop-up confirmation dialog box, select "SURE" to confirm the current operation, turn on or off the lamp, switch time interval limited to 30 seconds

	
	Off
	the current lamp output is off

	
	On
	The current lamp output is turned on


Scene mode applies to a single or a small number of fixture, just output a fixed scene, or need to run a simple program, you no need connect to the console, in the scene page can be edited.

If the light source is lamp, wait for 10 minutes before turning off the lamp.

Set display
[image: image11.png]English

Screen Rot
DMX Indicate

Screen Light





Figure 6-3  
The fixture support Chinese and English, invert display and so on. Enter the corresponding parameter settings as shown in Figure 6-3.The specific menu contents are as follows:

DISPLAY SETTING

	Language
	display language settings

	
	English
	English display

	
	Chinese
	Chinese display

	Screen saver
	Set screen 30 seconds without operation, the screen's display content or method.

	
	OFF
	Keep the last operation page

	
	Mode1
	Black

	
	Mode2
	Black screen, showing the address code of the current fixture in the lower left corner.

	
	Mode3
	Display trademark information, address code and operation mode.

	
	Mode4
	Display trademark information, address code and operation mode,which lasts for 30 seconds ,black screen.

	Screen Rot
	Set the display direction of the screen.

	
	OFF
	No reverse display

	
	ON
	Reverse display

	DMX Indicate
	Set the indication mode of DMX signal indicator.

	
	Mode1
	When signal is bright, no signal is off.

	
	Mode2
	When signal is off, no signal is bright.

	
	Mode3
	When signal is flash, no signal is off.

	Screen Light
	Set the screen backlight for 10 seconds without operation

	
	1~10
	10 


Scene

Enter the page shown in Figure 6-4(The channel shown in the picture is only an example of the function, please refer to the channel table description in the next section for the specific channel table of this product), and the fixture enters the scene editing mode. For example,under this page,when the [Control Mode] option is turned off ,the fixture does not receive DMX console data, and the edited data will effect on the fixture immediately.When it turned on, the console signal is received and the console data is read and reflected on the corresponding channel display.
[image: image12.png]Scene Select 1]

Scene Time

Control Mode
01.Colour
02.Strobe
03.Dimmer





Figure 6-4  
The content of the page depends on the currently selected channel mode, and the channel content and order displayed are consistent with the fixture channel table. Through this page, you can edit 10 scenes, as shown in the following table:

SCENE MODE

	Scene Select
	Select the current operation scenario.

	
	1~10
	The 10 scenes sets the format

	Scene Time
	Sets the retention time of the current scene when it is automatic,the final time is determined by the scene time multiplier, unit in 0.1 seconds.

	
	0
	The current scene is not output in automatic scene output.

	
	1-255
	0..1s-25.5s

	Control Mode
	Choose whether to use the console to manipulate the settings data

	
	OFF
	It is not possible to control the console and set the data directly from the current interface

	
	ON
	Using console control, the console data comes first when setting, and the setting is invalid in the current interface

	1. PAN
	0-255
	Set up the data of each channel, and the contents and order of the display are one-to-one correspondence with the channel list of fixture.

	……
	0-255
	

	……
	0-255
	

	N. Function
	0-255
	


If the reset channel in the scene edits the effective reset data, the fixture will reset, but after reset, the corresponding reset channel value will automatically set 0, preventing multiple consecutive resets.

Looking at this page, you can get the current channel table slot of the fixture. For specific channel data, please refer to the detailed channel description.

Set light run parameter
[image: image13.png]Tilt Invert

P/T Rectify
Pan Offset
Tilt Offset
Data hold





Figure 6-5
Enter the page shown in Figure 6-5, adjust the field parameters of fixture, facilitate the installation of fixture, etc.

ADVANCED SETTING

	Pan Invert
	Set the rotation direction of PAN

	
	OFF
	

	
	ON
	

	Tilt Invert
	Set the rotation direction of TILT

	
	OFF
	

	
	ON
	

	P/T Rectify
	Setting up fixture to detect XY lost step and correct

	
	OFF
	Uncorrected position after out of step

	
	ON
	After losing step, the position is automatically corrected and the out of step fault is recorded.

	Pan Offset
	Setting the zero point of the PAN of the fixture

	
	4-150
	　

	Tilt Offset
	Setting the zero point of the TILT of the fixture

	
	4-48
	　

	Data hold
	When the fixture is not equipped with DMX signal, the output state of the fixture

	
	OFF
	No signal, so the motor and light source return to the position and state when reset is completed.

	
	NO
	No signal, keep the last frame DMX data output.

	Scene Time

(multiple)
	Work with the scene time to determine the scene retention time

	
	1-255
	Retention time = Scene time * multiple

	Lamp mode
	(lamp light source) Set the way to first open the lamp after power up

	
	Power on
	Turn on the lamp at power up and reset the lamp after 30 seconds.

	
	After reset
	Reset the fixture after 3 seconds when power-on, and turn on the lamp after reset.

	
	Manual
	After reset, manually turn on the lamp through the menu or console.

	Reset
	Pop up the confirmation box, select "SURE", and reset the fixture.

	Factory Setting
	Pop up the confirmation box, select "SURE", and return the lamp parameters to the factory settings.


When choosing power-on mode, the lamp will wait for 30 seconds after power-on, let the lamp fully start, internal voltage is stable enough, then start the reset program, if the field capacity is stable, recommend power-on mode.

When the fixture can not calibrate the position, please check whether the "P/T Rectify" is turned off.

When the signal is unplugged, check the Data Hold setting first if the position of the fixture is not output as expected.

When setting the XY offset, after setting up, please control XY with the maximum stroke first to check that XY will not bump into the positioning rod or shell.

Status and information

[image: image14.png]Error Logging

Fixture Status
Version

Light time
Total time





Figure 6-6 
Entering the page shown in Figure 6-6, you can view the information and real-time status of the fixture to get their usage status. If the fixture need customer service, please provide the status information displayed on the page as a basis for judgment, as shown in the following table:

STATUS INFORMATION

	Stepper info
	Display information status of all motors and signals in fixture.

	
	Hall
	No display, indicating that the motor has no Hall, 0 indicating that the motor leaves the correction position point, 1 indicating that the motor is in the correction position point

	
	Status
	Display motor reset status

	
	PAN
	Display real-time position value of PAN optocoupler feedback

	
	TILT
	Display real-time position value of TILT optocoupler feedback

	
	PAN OP
	Displays the PAN TILT optocoupler two signal level state, binary

	Error Logging
	Show the latest 8 error records when the fixture is reset and running. The error records are not saved after power failure. The current power cycle is valid.

	
	Error Logging
	Total number of failures detected after power on

	
	12：:03
	The time of power failure when the fault occurs is in minutes.

	
	Hall error
	The effective hall signal is not detected when the motor is reset

	
	Hall short
	When the motor is reset, the hall signal of the motor is always effective

	
	Opti error
	No effective optocoupler signal is detected when the motor is reset.

	
	Lose stop
	The corresponding motor is out of step during its operation.

	
	Hit
	Striking the positioning rod when the motor is reset

	
	Lamp error
	Lamp explosion accident

	
	NTC error
	The temperature sensor signal is abnormal

	
	Fan error
	The main fan is not working properly.

	Fixture status
	Displays the critical state data of the current fixture for reference.

	
	Communication prec
	0~100%, Communication quality of internal data link of lamps and lanterns

	
	Error Cnt
	The number of erroneous frames was detected after power on, and the total number of erroneous frames was detected.

	
	Light Temperature
	Show the temperature of the current light source, "---" means no detection.

	
	Panel Temperature
	Displays the temperature of the current display panel or the ambient temperature.

	
	Sensor1 Temperature
	Display the ambient temperature of the motherboard temperature or the motherboard installation position.

	Version
	Display the information and version of the current fixture, important reference for after sales maintenance.

	
	Device
	The name of the fixture is the same as the equipment information of RDM.

	
	Model
	The type of fixture is the same as the model information of RDM.

	
	Panel
	Firmware version and serial number of display panel

	
	Main Board
	Firmware version and serial number of mother board 1

	Light time
	Record the total cumulative time of light source opening, unit minute, user manual cleaning, as a reference for regular maintenance of light source time.

	Total time
	The total accumulated time for recording the opening of fixture is not allowed to be removed.


Chapter 3  Channel description

Channel table
Note: the channel tables of different lamps are different. The following channel tables are for reference only

This luminance channel can be viewed in scene mode in order, channel mode is set in the "Address Settings" page, specific details of the data as follows:
CHANNEL TABLE

	LIST-1
	LIST-2
	NAME
	VALUE
	BRIEF

	[ CH1 ]
	[ CH1 ]
	Colour
	
	

	
	
	
	0-4
	White

	
	
	
	5-9
	White+colour1

	
	
	
	10-14
	Colour1

	
	
	
	15-19
	Colour1+Colour2

	
	
	
	20-24
	Colour2

	
	
	
	25-29
	Colour2+Colour3

	
	
	
	30-34
	Colour3

	
	
	
	35-39
	Colour3+Colour4

	
	
	
	40-44
	Colour4

	
	
	
	45-49
	Colour4+Colour5

	
	
	
	50-54
	Colour5

	
	
	
	55-59
	Colour5+Colour6

	
	
	
	60-64
	Colour6

	
	
	
	65-69
	Colour6+Colour7

	
	
	
	70-74
	Colour7

	
	
	
	75-79
	Colour7+Colour8

	
	
	
	80-84
	Colour8

	
	
	
	85-89
	Colour8+Colour9

	
	
	
	90-94
	Colour9

	
	
	
	95-99
	Colour9+Colour10

	
	
	
	100-104
	Colour10

	
	
	
	105-109
	Colour10+Colour11

	
	
	
	110-114
	Colour11

	
	
	
	115-119
	Colour11+Colour12

	
	
	
	120-124
	Colour12

	
	
	
	125-129
	Colour12+Colour13

	
	
	
	130-134
	Colour13

	
	
	
	135-139
	Colour13+Colour14

	
	
	
	140-197
	Rotate forward (fast to slow)

	
	
	
	198-255
	Rotate reverse (slow to fast)

	[ CH2 ]
	[ CH2 ]
	Strobe
	
	

	
	
	
	0-3
	Dark

	
	
	
	4-103
	Pluse strobe slow to fast

	
	
	
	104-107
	Open

	
	
	
	108-207
	Fade strobe slow to fast

	
	
	
	208-212
	Open

	
	
	
	213-251
	Rand strobe slow to fast

	
	
	
	252-255
	Open

	[ CH3 ]
	[ CH3 ]
	Dimmer
	0-255
	0-100% dimmer

	[ CH4 ]
	[ CH4 ]
	Gobo
	
	

	
	
	
	0-4
	White

	
	
	
	5-9
	Gobo1

	
	
	
	10-14
	Gobo2

	
	
	
	15-19
	Gobo3

	
	
	
	20-24
	Gobo4

	
	
	
	25-29
	Gobo5

	
	
	
	30-34
	Gobo6

	
	
	
	35-39
	Gobo7

	
	
	
	40-44
	Gobo8

	
	
	
	45-49
	Gobo9

	
	
	
	50-54
	Gobo10

	
	
	
	55-59
	Gobo11

	
	
	
	60-64
	Gobo12

	
	
	
	65-69
	Gobo13

	
	
	
	70-74
	Gobo14

	
	
	
	75-79
	Gobo15

	
	
	
	80-84
	Gobo16

	
	
	
	85-89
	Gobo17

	
	
	
	90-128
	Rotate reverse (fast to slow)

	
	
	
	129-131
	Stop

	
	
	
	132-170
	Rotate forward (slow to fast)

	
	
	
	171-175
	Shake slow to fast Gobo1

	
	
	
	176-180
	Shake slow to fast Gobo2

	
	
	
	181-185
	Shake slow to fast Gobo3

	
	
	
	186-190
	Shake slow to fast Gobo4

	
	
	
	191-195
	Shake slow to fast Gobo5

	
	
	
	196-200
	Shake slow to fast Gobo6

	
	
	
	201-205
	Shake slow to fast Gobo7

	
	
	
	206-210
	Shake slow to fast Gobo8

	
	
	
	211-215
	Shake slow to fast Gobo9

	
	
	
	216-220
	Shake slow to fast Gobo10

	
	
	
	221-225
	Shake slow to fast Gobo11

	
	
	
	226-230
	Shake slow to fast Gobo12

	
	
	
	231-235
	Shake slow to fast Gobo13

	
	
	
	236-240
	Shake slow to fast Gobo14

	
	
	
	241-245
	Shake slow to fast Gobo15

	
	
	
	246-250
	Shake slow to fast Gobo16

	
	
	
	251-255
	Shake slow to fast Gobo17

	[ CH5 ]
	[ CH5 ]
	Prism1
	
	

	
	
	
	0-127
	None

	
	
	
	128-255
	Inert prism1

	[ CH6 ]
	[ CH6 ]
	Prism1.R
	
	

	
	
	
	0-127
	0-360(degree)

	
	
	
	128-190
	Rotate forward (fast to slow)

	
	
	
	191-192
	Stop

	
	
	
	193-255
	Rotate reverse (slow to fast)

	[ CH7 ]
	[ CH7 ]
	Prism2
	
	

	
	
	
	0-127
	None

	
	
	
	128-255
	Insert prism2

	[ CH8 ]
	[ CH8 ]
	Frost
	
	

	
	
	
	0-127
	None

	
	
	
	128-191
	Insert colorful

	
	
	
	192-255
	Insert frost

	[ CH9 ]
	[ CH9 ]
	Focus
	0-255
	Far to near

	[ CH10 ]
	[ CH10 ]
	Pan
	0-255
	0-540(degree)

	[ CH11 ]
	[ CH11 ]
	Pan Fine
	0-255
	0-2(degree)

	[ CH12 ]
	[ CH12 ]
	Tilt
	0-255
	0-270(degree)

	[ CH13 ]
	[ CH13 ]
	Tilt Fine
	0-255
	0-1(degree)

	[ CH14 ]
	[ CH14 ]
	Macro
	0-255
	

	[ CH15 ]
	[ CH15 ]
	Reset
	
	

	
	
	
	0-25
	None

	
	
	
	26-76
	Reset Effect motor over 3 second

	
	
	
	77-127
	Reset XY motor over 3 second

	
	
	
	128-255
	Reset fxiture over 3 second

	[ CH16 ]
	[ CH16 ]
	Lamp
	
	

	
	
	
	0-25
	None

	
	
	
	26-100
	Turn off lamp over 3 second

	
	
	
	101-255
	Turn on over 3 second

	
	[ CH17 ]
	PT Spd
	0-255
	Fast to slow

	
	[ CH18 ]
	Color Spd
	0-255
	

	
	[ CH19 ]
	Focus Spd
	0-255
	

	
	[ CH20 ]
	Gobo Spd
	0-255
	Fast to slow

	[ CH17 ]
	[ CH21 ]
	Effect
	0-255
	

	[ CH18 ]
	[ CH22 ]
	Effect.Spd
	0-255
	




