12x40W RGBW 4in1 zoom wash pixel moving head light

User Manual

This manual contains important information. Please read before operating fixture.

1  SECURITY TIPS

WARNING!!! To reduce the risk of fire, electric shock, or injury to persons, follow these important safety instructions:

- non-professionals, not to disassemble the light arbitrarily and lighting accessories inside.

- rated voltage range: AC110V-120V/60Hz or AC200V-240V / 50HZ (to be selected).

- not beam irradiate the flammable materials, flammable materials and the lamp should be maintained at least 1M over the distance.

- lamps used in ambient temperature: -15 ℃ -40 ℃, the highest temperature of the surface of the lamp: 60 ℃.

- to keep away from the liquid substance and humid environment.

- before using the lights must ensure the good grounding, can not install charging and remove any parts.

- the installation of lamps, must be fixed screw fasteners with additional security cable and regular inspection.

- lamps consecutive working hours is not recommended more than 10 hours.

- should stop using the lights in time when happens unusual conditions during the process

- lamp parts of the rotation, external accessories and paste parts should be regular inspected, such as the emergence of loose and rock, should be reinforced in a timely manner to prevent the accident.

- lamps adopt strong winds to refrigerate, it is easy to accumulate the dust, so you have to clean it weekly especially the cooling air opening, otherwise the dust will plug leading to poor light

2  TECHNOLOGY REFERENCE

Input voltage: 110–240 V, 50/60 Hz

Power: 500W

Light source: 12pcs 40W, RGBW 4in1 mixing color

Working lifetime: About 50000 hours

IP rating: IP20

Working temperature:-20℃-40℃

Control mode: DMX 512, Auto, Master-slave, Sound

4Channel mode: STD/HSI/GUI/EXT (17/11/18/58CH)

Function: Dyeing, strobe, zoom, light beam effect

Pan: 540° (16bit) Electric correction

Tilt: 270° (16bit) Electric correction

Zoom range: 7°~ 70°

Dimmer: 0-100% dimmer

Display panel: Blue LCD screen

Housing material: Engineering plastics

Net weight: 15kg

MIN ZOOM-7° Beam Diameter

| Distance(m) | 3m | 5m | 7m |
| --- | --- | --- | --- |
| Beam Openning | 0.60m | 0.90m | xxm |
| Itensity(lux) | lux | lux | lux |
| Red | 10920 | 4900 | 2140 |
| Green | 22100 | 9038 | 4296 |
| Blue | 25800 | 9953 | 4300 |
| White | 29820 | 12560 | 6000 |
| R+G+B+W | 92100 | 23560 | 11510 |

MIN ZOOM-70°Beam Diameter

| Distance(m) | 1m | 3m | 5m |
| --- | --- | --- | --- |
| Beam Openning | XX | XX | XX |
| Itensity(lux) | lux | lux | lux |
| Red | 3536 | 518 | 198 |
| Green | 6343 | 855 | 335 |
| Blue | 6350 | 812 | 297 |
| White | 9248 | 1160 | 468 |
| R+G+B+W | 17580 | 2250 | 856 |

3  Board Instruction

| ADDRESS | 001-512 |  |
| --- | --- | --- |
| RUN | DMX512 |  |
|  | Auto |  |
| DMX MODE | 19CH |  |
|  | 67CH |  |
| TEST | ALL |  |
| SET | Display | Yes |
|  |  | No |
|  | Signal Keep | Yes |
|  |  | No |
|  | Slave | Yes |
|  |  | No |
|  | English | Yes |
|  |  | 否 |
| INFO | Version | V1.11 |
| MANUAL | Pan | 000-255 |
|  | Tilt | 000-255 |
|  | …… | …… |
|  | …… | …… |
|  | Strobe | 000-255 |
| RESET | Yes |  |
|  | No |  |

4  DMX CHANNEL SHEET

Mode:19CH

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan rotation |
| 2 | Pan Fine | 0-255 | Pan fine |
| 3 | Tilt | 0-255 | Tilt rotation |
| 4 | Tilt Fine | 0-255 | Tilt fine |
| 5 | P/T Speed | 0-255 | P/T speed |
| 6 | Dim | 0-255 | Total dimmer 0-100% |
| 7 | Zoom | 0-255 | Wide beam- narrow beam |
| 8 | Strobe | 0 - 10 | No function |
|  |  | 11-255 | Strobe,from slow to fast |
| 9 | Red | 0-255 | LED off - LED on |
| 10 | Green | 0-255 | LED off - LED on |
| 11 | Blue | 0-255 | LED off - LED on |
| 12 | White | 0-255 | LED off - LED on |
| 13 | CCT | 0-255 | colour temperature adjusting |
| 14 | Macro | 0-255 | Macro color |
| 15 | Effect | 0 | Closed |
|  |  | 1-100 | Dream,gradual change from slow to fast |
|  |  | 101-200 | Wave,from slow to fast |
|  |  | 201-255 | No function |
| 16 | Macro | 0-255 | Macro color |
| 17 | Background DIM | 0-255 | Background Dimmer |
| 18 | Background | 0-255 | Background color |
| 19 | Reset | 0-244 | No function |
|  |  | 245-255 | Fixture reset |

Mode:67CH

| Channel | Function | DMX Value | Description |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan rotation |
| 2 | Pan Fine | 0-255 | Pan fine |
| 3 | Tilt | 0-255 | Tilt rotation |
| 4 | Tilt Fine | 0-255 | Tilt fine |
| 5 | P/T Speed | 0-255 | P/T speed |
| 6 | Dim | 0-255 | Total dimmer 0-100% |
| 7 | Zoom | 0-255 | Wide beam- narrow beam |
| 8 | Strobe | 0 - 10 | No function |
|  |  | 11-255 | Strobe,from slow to fast |
| 9 | Red | 0-255 | LED off - LED on |
| 10 | Green | 0-255 | LED off - LED on |
| 11 | Blue | 0-255 | LED off - LED on |
| 12 | White | 0-255 | LED off - LED on |
| 13 | CCT | 0-255 | colour temperature adjusting |
| 14 | Macro | 0-255 | Macro color |
| 15 | Effect | 0 | Closed |
|  |  | 1-100 | Dream,gradual change from slow to fast |
|  |  | 101-200 | Wave,from slow to fast |
|  |  | 201-255 | No function |
| 16 | Macro | 0-255 | Macro color |
| 17 | Background DIM | 0-255 | Background Dimmer |
| 18 | Background | 0-255 | Background color |
| 19 | Reset | 0-244 | No function |
|  |  | 245-255 | Fixture reset |

| 20 | Red 1 | 0-255 | LED Red1 |
| --- | --- | --- | --- |
| 21 | Green 1 | 0-255 | LED Green 1 |
| 22 | Blue 1 | 0-255 | LED Blue 1 |
| 23 | White 1 | 0-255 | LED White 1 |
| 24 | Red 2 | 0-255 | LED Red 2 |
| 25 | Green 2 | 0-255 | LED Green 2 |
| 26 | Blue 2 | 0-255 | LED Blue 2 |
| 27 | White 2 | 0-255 | LED White 2 |
| 28 | Red 3 | 0-255 | LED Red 3 |
| 29 | Green 3 | 0-255 | LED Green3 |
| 30 | Blue 3 | 0-255 | LED Blue 3 |
| 31 | White 3 | 0-255 | LED White 3 |
| 32 | Red 4 | 0-255 | LED Red 4 |
| 33 | Green 4 | 0-255 | LED Green 4 |
| 34 | Blue 4 | 0-255 | LED Blue 4 |
| 35 | White 4 | 0-255 | LED White 4 |
| 36 | Red 5 | 0-255 | LED Red 5 |
| 37 | Green 5 | 0-255 | LED Green 5 |
| 38 | Blue 5 | 0-255 | LED Blue 5 |
| 39 | White 5 | 0-255 | LED White 5 |
| : | Red 6 | 0-255 | LED Red 6 |
| 64 | Red 12 | 0-255 | LED Red 12 |
| 65 | Green 12 | 0-255 | LED Green 12 |
| 66 | Blue 12 | 0-255 | LED Blue 12 |
| 67 | White 12 | 0-255 | LED White 12 |

5  DIFFICULTY RESOLUTION

The following is a list of problems and resolution, and other malfunction preparing work should be professionally qualified staff to deal with.

1. Bulb not working

  - Check whether approaching the service life of light bulbs, then change it

  - Check whether bulbs circuitry leak, loss or connect poorly

2. Beam appears bleak

- The bulbs maybe approach the service life, then change it, or the optical components are unclean.

- Bulbs have accumulation of dust, so the bulbs and other parts inside the lamp should be cleaned and maintained in regular period.

3. Computer lamp work intermittently

- Check whether the fan process naturally and become dirty

- Check inner temperature-response control plug is off

- Check whether the circuitry is good

4. The computer lamp not controlled by controller during lighting

- Check numeral starting address and the connection of communication control circuit

- The corresponding channel button of controller is open or not

- Check whether the fuse on power supply input jack is broken off

6  AFTER SERVICE

- Our company offer technical consultation to customers for ever.

- If the machine is wrong and need repairmen, please truthfully fill in the related content and feedback us the problems timely, as well as we have to improve the product.

- In addition to the lamp, also matching accessories are as follows：

accessories：Power line  1 piece    Optional parts：(customers）