Laser Show System

User Guide

User Manual

Like above picture，

MODE：Press one time change one mode

UP、DOWN：when in PRG and IL mode short press to change the working file, long press to change the whole  folder, when in sound mode, it’ll change the acoustic sensitivity.

matters needing attention

1: The optimum temperature range of laser lamp is 20 ~ 35 ℃.

2: Do not direct the laser at human eyes to avoid injury.

3: Please do not use beyond AC110V ~ 240V voltage, and ensure reliable grounding

4: Please do not switch frequently, so as not to affect the normal service life of the laser lamp.

5: Do not touch the lens of the laser projection window with your hand, so as not to affect the use effect.

6: In case of failure, please ask professional personnel for maintenance. Do not disassemble the lamp without authorization.

7: According to the use environment, clean the lens regularly to ensure the light efficiency

Please note: artificial damage or tear warranty label, are not within the scope of warranty! Please refer to the warranty details

Power consumption table of laser lamp:

| Output power / W | Total power（W） |  |
| --- | --- | --- |
|  | Min | Max |
| RGB/6W | 40 | 70 |
| RGB/10W | 60 | 110 |
| RGB/12W | 70 | 130 |
| RGB/15W | 80 | 140 |
| RGB/20W | 95 | 220 |
| RGB/30W | 310 | 520 |
| RGB/40W | 370 | 580 |
| RGB/50W | 410 | 670 |
| RGB/60W | 480 | 720 |

Laser range and coverage:

The projection range is a square size of H* H, as shown in the figure

The length of H = projection distance * 0.93

0.93 is the scanning coefficient of plus or minus 25 degrees. If the scanning parameter is plus or minus 20 degrees, the coefficient needs to be changed to 0.73

Operating instructions: display screen and corresponding functions

| Operation must see: the button can be pressed or rotated left and right. When the screen is black: press the button or rotate the button once to light up the screen to view the current state, and then press once to enter the setting state. After setting, press once to turn off the blue cursor, and then double-click the button to save |  |  |  |
| --- | --- | --- | --- |
| display | parameter | function | You have to read it |
| Dmx Address | 0~512 | DMX512 MODE | Press the button once, the place to be set will become a blue cursor. Rotate the button to change the parameters. When the required parameters are reached, press once to turn off the blue cursor. After double clicking twice to save, it will exit automatically |
| Show mode | ILD | Play a single animation |  |
|  | Sound | Sound mode |  |
|  | Auto | Auto mode |  |
|  | PRG | Play the animation in order |  |
| Program | Program1 |  |  |
|  | Program2 |  |  |
|  | Program3 |  |  |
| SIZE | 10~100 | Drawing size settings |  |
| Phasic set | X+ Y+ | Default X and Y directions) |  |
|  | X- Y+ | X phase inversion |  |
|  | X- Y- | X phase inversion both X and Y phase inversion |  |
|  | X+ Y- | Y phase inversion |  |
| Speed set | 15~40 | 25 for 10W and 30 for 20W |  |
| DMX STATE | Show Mode | When there is no signal, play according to the set mode |  |
|  | Black out | Turn off the light and stand by when there is no signal |  |
| SLAVE MODE | Slave | Follow host |  |
|  | Maste | Host mode |  |
| X Phasic | This setting changes both the local and external ILDA input phases |  |  |
|  | Positive | X Normal phase |  |
|  | Reverse | X Inverse phase |  |
| Y Phasic | Positive | Y Normal phase |  |
|  | Reverse | Y Inverse phase |  |
| Laser Lock | On | Turn off the light when doing dot |  |
|  | Off | Do not close the light when you do the dot The lamp above 20W has this function |  |
| Soundsense | 0-100 | Voice sensitivity setting |  |
| SD soun | On/ off | Whether to turn on voice control in animation mode |  |
| Language 语言 | English | English display |  |
|  | 中文 | Chinese display |  |
| R | 0~100 | Red brightness |  |
| G | 0~100 | Green brightness |  |
| B | 0~100 | Blue brightness |  |
| LOAD FLASH | on~ 0ff | Non terminal function (not available) |  |
| Color | RGB/W | Full color play and monochrome play conversion |  |
| Pointment luminance | 0~10 | Scan delay dwell wait |  |
| CH Mode | 18CH/29CH | 18CH/29CH |  |

DMX512  18CH

| Channel | Function | Value | Description |
| --- | --- | --- | --- |
| CH1 | Dimming | 0~255 | From dark to light |
| CH2 | MODE | 0~49 | Auto）The effect inside the lamp plays automatically |
|  |  | 50~99 | Sound）The effect inside the lamp is played by sound |
|  |  | 100~149 | Sequence of animation programs |
|  |  | 150~199 | Single animation effect playing |
|  |  | 200~255 | Manual mode （Program change via CH3） |
| CH3 | Change program | 0~255 | Change program |
| CH4 | Stroboscopic | 0~10 | Turn off strobe |
|  |  | 11~255 | Auto strobe, speed from slow to fast |
| CH5 | Color | 0~16 | white |
|  |  | 17~33 | Red |
|  |  | 34~50 | Green |
|  |  | 51~67 | Blue |
|  |  | 68~84 | Yellow |
|  |  | 85~101 | Purple |
|  |  | 102~118 | Cyan |
|  |  | 119~135 | White, red, green, blue color section |
|  |  | 136~152 | Blue, yellow, purple, cyan color section |
|  |  | 153~169 | W, R, G, B, Y, P, C 7 color section |
|  |  | 170~186 | White, red, green, blue 4 color flow |
|  |  | 187~203 | Blue, yellow, purple, cyan 4 color flow |
|  |  | 204~220 | Blue, yellow, purple, cyan 4 color flow |
|  |  | 221~237 | color subsection by inflexion |
|  |  | 238~255 | Sound active color change |
| CH6 | Display Mode | 0~63 | Normal display |
|  |  | 64~127 | Light dot display |
|  |  | 128~191 | Segment display |
|  |  | 192~255 | Dot display |
| CH7 | X move | 0~125 | Adjust position by manual |
|  |  | 126~185 | Move circle from left to right automatically |
|  |  | 186~225 | Jump circle from right to left automatically |
|  |  | 226~245 | Auto jumping |
|  |  | 246~255 | Audio jumping |
| CH8 | Y move | 0~125 | Adjust position by manual |
|  |  | 126~185 | Move circle from up to down automatically |
|  |  | 186~225 | Jump circle from down to up automatically |
|  |  | 226~245 | Auto jumping |
|  |  | 246~255 | Audio jumping |
| CH9 | Zoom(+/-) | 0~10 | No change |
|  |  | 11~87 | Adjust size by manual |
|  |  | 88~150 | Zoom + |
|  |  | 151~200 | Zoom - |
|  |  | 201~255 | Zoom (+/-) circle |
| CH10 | Rolling X | 0 | No change |
|  |  | 1~128 | Manual rotation |
|  |  | 129~255 | Auto rotation |
| CH11 | Rolling Y | 0 | No change |
|  |  | 1~128 | Manual rotation |
|  |  | 129~255 | Auto rotation |
| CH12 | Rolling Center | 0 | No change |
|  |  | 1~128 | Manual rotation |
|  |  | 129~192 | Auto clockwise rotation |
|  |  | 193~255 | Auto counterclockwise rotation |
| CH13 | Drawing | 0~10 | No change |
|  |  | 10~74 | Manual drawing |
|  |  | 75~104 | Auto drawing + |
|  |  | 105~144 | Auto drawing - |
|  |  | 145~184 | Auto drawing circle |
|  |  | 185~224 | End to end drawing circle + |
|  |  | 225~255 | End to end drawing circle - |
| CH14 | X wave | 0~9 | No wave |
|  |  | 10~69 | Small wave |
|  |  | 70~129 | Medium wave |
|  |  | 130~189 | Big wave |
|  |  | 190~255 | Biggest wave |
| CH15 | Y wave | 0~9 | No wave |
|  |  | 10~69 | Small wave |
|  |  | 70~129 | Medium wave |
|  |  | 130~189 | Big wave |
|  |  | 190~255 | Biggest wave |
| CH16 | Red Dimmer | 0~255 | From darkest to brightest |
| CH17 | Green Dimmer | 0~255 | From darkest to brightest |
| CH18 | Blue Dimmer | 0~255 | From darkest to brightest |

DMX512  29CH

| Channel | Function | Value | Description |
| --- | --- | --- | --- |
| CH1 | Dimming | 0~255 | From darkest to brightest |
| CH2 | MODE | 0~49 | Auto）The effect inside the lamp plays automatically |
|  |  | 50~99 | Sound）The effect inside the lamp is played by sound |
|  |  | 100~149 | Sequence of animation programs |
|  |  | 150~199 | Single animation effect playing |
|  |  | 200~255 | Manual mode （Program change via CH3） |
| CH3 | Change program | 0~255 | Change program |
| CH4 | Stroboscopic | 0~10 | Turn off strobe |
|  |  | 11~255 | Auto strobe, speed from slow to fast |
| CH5 | Red Dimmer | 0~255 | From darkest to brightest |
| CH6 | Green Dimmer | 0~255 | From darkest to brightest |
| CH7 | Blue Dimmer | 0~255 | From darkest to brightest |
| CH8 | Color | 0~16 | white |
|  |  | 17~33 | Red |
|  |  | 34~50 | Green |
|  |  | 51~67 | Blue |
|  |  | 68~84 | Yellow |
|  |  | 85~101 | Purple |
|  |  | 102~118 | Cyan |
|  |  | 119~135 | White, red, green, blue color section |
|  |  | 136~152 | Blue, yellow, purple, cyan color section |
|  |  | 153~169 | W, R, G, B, Y, P, C 7 color section |
|  |  | 170~186 | White, red, green, blue 4 color flow |
|  |  | 187~203 | Blue, yellow, purple, cyan 4 color flow |
|  |  | 204~220 | Blue, yellow, purple, cyan 4 color flow |
|  |  | 221~237 | color subsection by inflexion |
|  |  | 238~255 | Sound active color change |
| CH9 | Graphics out of bounds effect | 0~49 | Cut after the graph is out of bounds |
|  |  | 50~99 | After the graph is out of bounds, the out of bounds part is displayed on the opposite side |
|  |  | 100~149 | After the graph goes beyond the boundary, it bounces back |
|  |  | 150~255 | After the graph goes beyond the boundary, it is compressed into lines at the boundary |
| CH10 | X position | 0~255 | Auto zoom manually adjust x position (valid only when CH19 < 2) |
| CH11 | X Fine tuning | 0~255 | Manual adjustment of X trim (effective only when CH19 < 2) |
| CH12 | Y position | 0~255 | Manually adjust y position (valid only when ch20 < 2) |
| CH13 | Y Fine tuning | 0~255 | Manually adjust y trim (valid only when ch20 < 2) |
| CH14 | Zoom(+/-) | 0~255 | Manually adjust the size of graphics (valid only when ch21 < 11) |
| CH15 | X angle | 0~255 | Manually adjust the X angle (valid only when ch22 = 0) |
| CH16 | Y angle | 0~255 | Manually adjust the Y angle (valid only when ch23 = 0) |
| CH17 | Z angle | 0~255 | Manually adjust Z angle (valid only when ch24 < 2) |
| CH18 | Gradually drawing | 0~255 | Manual adjustment of gradients |
| CH19 | X Position macro motion X auto move | 0~1 | Manual movement (ch10 valid) |
|  |  | 1~100 | Automatically move from right to left, the faster the value |
|  |  | 101~200 | Automatically move from left to right, the faster the value |
|  |  | 201~245 | Left and right automatic cycle beating, the faster the value |
|  |  | 246~255 | Left and right cycle beating triggered by voice control |
| CH20 | Y Position macro motion Y auto move | 0~1 | Manual movement (CH12 valid) |
|  |  | 2~100 | Automatically move from bottom to top, the faster the value |
|  |  | 101~200 | Automatically move from top to bottom, the faster the value |
|  |  | 201~245 | Up and down automatic cycle beating, the faster the value |
|  |  | 246~255 | Up and down cycle jumping triggered by voice control |
| CH21 | Figure size macro motion（Auto zoom） | 0~10 | Manual zoom (ch14 valid) |
|  |  | 11~80 | Auto zoom in, the faster the value |
|  |  | 81~160 | Auto shrink, the faster the value |
|  |  | 281~255 | Auto cycle scaling, the larger the value, the faster |
| CH22 | X angle macro motion | 0 | X manual flip (CH15 valid) |
|  |  | 1~255 | Auto flip from slow to fast |
| CH23 | Y angle macro motion | 0 | Y manual flip (ch16 valid) |
|  |  | 1~255 | Auto flip from slow to fast |
| CH24 | Z angle macro motion | 0~2 | Manual rotation (ch17 active) |
|  |  | 3~128 | Counter clockwise automatic rotation, speed up gradually |
|  |  | 129~130 | No rotation |
|  |  | 131~255 | Clockwise automatic rotation, speed up gradually |
| CH25 | Progressive macro motion | 0~74 | Manual adjustment of gradients |
|  |  | 75~104 | Auto fade (add) |
|  |  | 105~144 | Auto fade (subtract) |
|  |  | 145~184 | Auto loop graduals |
|  |  | 185~224 | End to end circular gradual drawing (increase) |
|  |  | 225~255 | End to end loop progressive drawing (subtraction) |
| CH26 | X wave macro motion | 0~10 | There are no waves |
|  |  | 11~69 | Small wave, speed gradually accelerated |
|  |  | 70~129 | Half wave |
|  |  | 130~189 | Big wave |
|  |  | 190~255 | Super wave |
| CH27 | Y wave macro motion | 0~10 | There are no waves |
|  |  | 11~69 | Small wave, speed gradually accelerated |
|  |  | 70~129 | Half wave |
|  |  | 130~189 | Big wave |
|  |  | 190~255 | Super wave |
| CH28 | Wave size adjustment | 0~255 | Waves from big to small |
| CH29 | Display effect | 0~63 | Default value |
|  |  | 64~127 | Add highlights |
|  |  | 128~191 | Line segment display |
|  |  | 192~255 | Point display |