I. Display panel and key button definition

Menu, Up, Down, confirm

Food menu key: Select the function

Up key:, parameter arguments

Down key:, parameter decline

Confirm key: Determine and save

2. Menu function

Press the menu key, then the menu table; press up or down to modify the function parameters and confirm that the key saves the current function and parameters (power memory after saving).

menu function table:

| A001 |  | A512 | Modify the address code up or down (A001~A 512) to confirm that the key is saved, with the default of A001. |
| --- | --- | --- | --- |
| CH04 |  | CH72 | Switch the CH04, CH08, and CH72 channels up or down to confirm that the key is saved, with the default CH08. |
| M000 |  | M126 | Modify the built-in effect up or down, confirm the key save, default M000. |
| S000 |  | S255 | Modify the built-in effect running speed (S000-S 255) and confirm that the key is saved, with the default S 000. |
| R255 |  | R000 | Modify the red bead brightness (R 000-R255) up or down, and confirm that the key is saved, with the default R 255. |
| G255 |  | G000 | Modify the green bead brightness (G000~ G255) to confirm that the key is saved, with the default G255. |
| B255 |  | B000 | Modify the blue bead brightness up or down (B000~ B255), and confirm that the key is saved, with the default B255. |
| W255 |  | W000 | Modify the white bead brightness up or down (W000~ W255), and confirm that the key is saved, with the default W255. |
| T000 |  |  | Display temperature, such as T045 indicates the current lamp temperature of 45℃; 10K thermistor is not installed, display T000. |

Iii. Factory Settings

At any address code from A 001 to A 512, press the menu key for 3 seconds to enter the factory setting.Factory setting is mainly the function of output power per lamp, fan setting mode, setting temperature protection point and sending parameters. The factory sets any mode according to the single key for 3 seconds.

Factory Settings Mode Table:

| R 255 |  | R032 | Modify the red bead current (R032-R 255) up or down to confirm that the key is saved, with the default R220. |
| --- | --- | --- | --- |
| G 255 |  | G032 | Modify the green bead current up or down (G 032-G255) to confirm that the key is saved, with the default G220. |
| B 255 |  | B032 | Modify the blue bead current up or down (B 032-B255), and confirm that the key is saved with the default B220. |
| W255 |  | W032 | Modify the white bead current (W032~ W255) up or down to confirm that the key is saved with the default W220. |
| FAN0 |  | FAN1 | Fan setting: FAN0 starts the fan, FAN1 reaches the set temperature protection point to start the fan, and confirm the key preservation. |
| T040 |  | T 070 | Modify temperature parameters (40 down (40-70℃) and press OK to save. |
| S end |  | S end | Send the parameters set by the local factory up or down to all other lamps connected by three-core signal lines; confirm that the transmission parameters exit by the dish menu key for 3 seconds, and cancel the transmission by the confirmation key. |

IV. Main and slave machine control

Two or more identical lamps are connected with DMX three-core signal line, all lamps are set to A001~A512 any address code, set as the host, and other lamps adjust the gradient, pulse, jump and self-departure effects, all slave synchronous gradient, pulse, jump and self-walk effects.

Special attention: 1. A group of lamps can only set up one host. If there are multiple hosts, all the lamps will flash and out of time.

2. All lamps must be the master when the DMX512 console is turned off.

V. DMX512 console

After power, all lamp address codes will be set, and then all lamps will be connected to the DMX512 console in parallel with three-core signal lines, and the address code will stop flashing, indicating that the DMX512 console signal has been sent to the lamp, and relevant functions will be controlled with the DMX512 console according to each channel description.

CH04 channel Description:

| channel | The channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | Red lamp beads for linear dimming. |
| 2 | 000-255 | Green lamp beads for linear dimming. |
| 3 | 000-255 | Blue lamp beads for linear dimming. |
| 4 | 000-255 | White lamp beads for linear dimming. |

CH08-channel Description:

| channel | The channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | stroboflash |
| 3 | 000-255 | Mode, the specific effect to see: six, mode effect. |
| 4 | 000-255 | velocity |
| 5 | 000-255 | Red lamp beads for linear dimming. |
| 6 | 000-255 | Green lamp beads for linear dimming. |
| 7 | 000-255 | Blue lamp beads for linear dimming. |
| 8 | 000-255 | White lamp beads for linear dimming. |

CH72-channel Description:

| channel | The channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | The 1st red lamp bead has a linear dimming. |
| 2 | 000-255 | The 1st green light bead has a linear dimming. |
| 3 | 000-255 | The 1st blue light bead has a linear dimming. |
| 4 | 000-255 | The 1st white lamp bead has a linear dimming. |
| … | … | …… |
| 69 | 000-255 | The 18th red lamp bead has a linear dimming. |
| 70 | 000-255 | The 18th green light bead has a linear dimming. |
| 71 | 000-255 | The 18th blue light bead has a linear dimming. |
| 72 | 000-255 | The 18th white light bead has a linear dimming. |

VI. Mode effect (prompt: mode code 11~114, which can push and pull RGB to change the background color.）

| The channel value | Mode code name | effect |
| --- | --- | --- |
| 0-1 | 0 | inaction |
| 2-3 | 1 | R red lamp. |
| 4-5 | 2 | G green light. |
| 6-7 | 3 | B blue light. |
| 8-9 | 4 | W white lamp. |
| 10-11 | 5 | RG red and green stained lamp. |
| 12-13 | 6 | RB red and blue stained light. |
| 14-15 | 7 | GB green and blue stained light. |
| 16-17 | 8 | Comprehensive 1-7 effect cycle. |
| 18-19 | 9 | gradual change |
| 20-21 | 10 | Pulse change |
| 22-23 | 11 | A red light for horse racing. |
| 24-25 | 12 | A green light for horse racing. |
| 26-27 | 13 | A blue light for horse racing. |
| 28-29 | 14 | A white light for horse racing. |
| 30-31 | 15 | A red and green stained light for horse racing. |
| 32-33 | 16 | A red and blue stained light for horse racing. |
| 34-35 | 17 | A green and blue stained light for horse racing. |
| 36-37 | 18 | Comprehensive 11-17 effect cycle. |
| 38-39 | 19 | A red light and a green light are running on a horse. |
| 40-41 | 20 | A green light and a blue light for horse racing. |
| 42-43 | 21 | A blue light and a white light for horse racing. |
| 44-45 | 22 | A white light and a red and green stained light racing horse. |
| 46-47 | 23 | A red and green stained light and a red and blue stained light for horse racing. |
| 48-49 | 24 | A red and blue stained light and a green and blue stained light are horse racing. |
| 50-51 | 25 | A green and blue stained lamp and a red light are racing on a horse. |
| 52-53 | 26 | Comprehensive 19-25 effect cycle. |
| 54-55 | 27 | A red light, a green light, and a blue light for horse racing. |
| 56-57 | 28 | A green light, a blue light, and a white light for horse racing. |
| 58-59 | 29 | A blue light, a white light, and a red and green stained light ran horses. |
| 60-61 | 30 | A white light, a red and green stained light, and a red and blue stained light ran on horses. |
| 62-63 | 31 | A red and green stained light, a red and blue stained light, and a green and blue stained light ran horses. |
| 64-65 | 32 | A red and blue stained light, a green and blue stained lamp and a red light racing horse. |
| 66-67 | 33 | A green and blue stained lamp, a red lamp, and a green light ran a horse. |
| 68-69 | 34 | Comprehensive 27-33 effect cycle. |
| 70-71 | 35 | A red light is refreshed. |
| 72-73 | 36 | A green light is refreshed. |
| 74-75 | 37 | A blue light is refreshed. |
| 76-77 | 38 | A white light is refreshed. |
| 78-79 | 39 | A red and green stained light was refreshed. |
| 80-81 | 40 | A red and blue stained light was refreshed. |
| 82-83 | 41 | A green and blue stained light was refreshed. |
| 84-85 | 42 | Comprehensive 35-41 effect cycle. |
| 86-87 | 43 | Two red lights to refresh. |
| 88-89 | 44 | Two green lights to refresh. |
| 90-91 | 45 | Two blue lights to refresh. |
| 92-93 | 46 | Two white lights to refresh. |
| 94-95 | 47 | Two red and green stained lights were refreshed. |
| 96-97 | 48 | Two red and blue stained lights were refreshed. |
| 98-99 | 49 | Two green and blue stained lights were refreshed. |
| 100-101 | 50 | Comprehensive 43-49 effect cycle. |
| 102-103 | 51 | A red light runs back and forth. |
| 104-105 | 52 | A green light runs back and forth. |
| 106-107 | 53 | A blue light ran back and forth. |
| 108-109 | 54 | A white light ran back and forth. |
| 110-111 | 55 | A red and green stained lamp ran back and forth. |
| 112-113 | 56 | A red and blue-stained light ran back and forth. |
| 114-115 | 57 | A green and blue-stained lamp ran back and forth. |
| 116-117 | 58 | Comprehensive 51-57 effect cycle. |
| 118-119 | 59 | Two red lights run back and forth. |
| 120-121 | 60 | Two green lights run back and forth. |
| 122-123 | 61 | Two blue lights run back and forth. |
| 124-125 | 62 | Two white lights ran back and forth. |
| 126-127 | 63 | Two red and green stained lights run back and forth. |
| 128-129 | 64 | Two red and blue stained lights ran back and forth. |
| 130-131 | 65 | Two green and blue stained lights run back and forth. |
| 132-133 | 66 | Comprehensive 59-65 effect cycle. |
| 134-135 | 67 | One red light at each end runs back and forth. |
| 136-137 | 68 | One green light at each end runs back and forth. |
| 138-139 | 69 | One blue light at each end ran back and forth. |
| 140-141 | 70 | One white light at each end runs back and forth. |
| 142-143 | 71 | One red and green stained light at each end runs back and forth. |
| 144-145 | 72 | One red and blue stained light at each end ran back and forth. |
| 146-147 | 73 | One green and blue stained lamp at each end runs back and forth. |
| 148-149 | 74 | Comprehensive 67-73 effect cycle. |
| 150-151 | 75 | Two red lights at each ran back and forth. |
| 152-153 | 76 | Two green lights at each end run back and forth. |
| 154-155 | 77 | Two blue lights run back and forth. |
| 156-157 | 78 | Two white lights at each end run back and forth. |
| 158-159 | 79 | Two red and green stained lights at each end ran back and forth. |
| 160-161 | 80 | Two red and blue stained lights ran back and forth. |
| 162-163 | 81 | Two green and blue stained lights run back and forth. |
| 164-165 | 82 | Comprehensive 75-81 effect cycle. |
| 166-167 | 83 | Red light meteor shower. |
| 168-169 | 84 | Green light meteor shower. |
| 170-171 | 85 | Blue light meteor shower. |
| 172-173 | 86 | White light meteor shower. |
| 174-175 | 87 | Red and green stained lamp meteor shower. |
| 176-177 | 88 | Red and blue stained lamp meteor shower. |
| 178-179 | 89 | Green and blue stained lamp meteor shower. |
| 180-181 | 90 | Comprehensive 83-89 effect cycle. |
| 182-183 | 91 | A red light horse racing has a residual shadow. |
| 184-185 | 92 | A green light horse residual shadow. |
| 186-187 | 93 | A blue light horse racing has a residual shadow. |
| 188-189 | 94 | A white light horse racing with a residual shadow. |
| 190-191 | 95 | A red and green stained light horse racing with a residual shadow. |
| 192-193 | 96 | A red and blue stained lamp horse racing has a residual shadow. |
| 194-195 | 97 | A green and blue stained lamp horse racing has a residual shadow. |
| 196-197 | 98 | Comprehensive 91-97 effect cycle. |
| 198-199 | 99 | Two red light pendulum. |
| 200-201 | 100 | Two green light pendulum. |
| 202-203 | 101 | Two blue light pendulum. |
| 204-205 | 102 | Two white light pendulum. |
| 206-207 | 103 | Two red and green stained light pendulum. |
| 208-209 | 104 | Two red and blue stained light pendulum. |
| 210-211 | 105 | Two green and blue stained light pendulum. |
| 212-213 | 106 | Comprehensive 99-105 effect cycle. |
| 214-215 | 107 | A red light was piled up. |
| 216-217 | 108 | A green light is piled up. |
| 218-219 | 109 | A blue light was piled up. |
| 220-221 | 110 | A white light was piled up. |
| 222-223 | 111 | A red and green stained lamp piled up. |
| 224-225 | 112 | A red and blue stained lamp was piled up. |
| 226-227 | 113 | A green and blue-stained lamp was piled up. |
| 228-229 | 114 | Comprehensive 107-113 effect cycle. |
| 230-231 | 115 | Red waves. |
| 232-233 | 116 | Green waves. |
| 234-235 | 117 | Blue waves. |
| 236-237 | 118 | White waves. |
| 238-239 | 119 | Red and green stained waves. |
| 240-241 | 120 | Red and blue-stained waves. |
| 242-243 | 121 | Green and blue-stained waves. |
| 244-245 | 122 | Comprehensive 115-121 effect cycle. |
| 246-247 | 123 | Colorful water. |
| 248-249 | 124 | Colorful refresh, from the middle to both sides, a color on one side. |
| 250-251 | 125 | Colorful from the middle to both sides of the color color refresh. |
| 252-253 | 126 | Colorful refresh, refresh on one color for another color. |
| 254-255 | 127 | Mode code name 11-126 cycle. |

VII. Technical parameters:

Voltage: AC100~240V 50 / 60HZ

power:,80W

Beads: 18 four-in-one LED beads

Control mode: DMX512, self-walk, main and slave, with RDM function.

Channel: CH04, CH08, CH72.

Dimming: 32bit 0~100% linear dimming

Features: dyeing + explosive flash + wall washing lamp

Operating temperature: -30 degrees ~50 degrees

Strobe frequency: 1~30HZ

Appearance: Metal, black

Connection mode: DMX512 input / output / power input / output.

IP level: IP20

size:  weight: