Full color animated laser light

an instruction manual

Note: This manual contains important information on how to install and use it safely. Please read it carefully and install and operate as required. At the same time, please keep this manual properly for emergencies

- Product Notes:

- 1. In order to ensure the service life of the product, the product should not be placed in a wet or leaky place, not to mention.

- 1 can work in an environment with temperature over 60 degrees

- 2. Do not place the product in a place that is easy to loosen or shake.

- 3. In order to avoid the danger of electric shock, the maintenance request of this product helps professional maintenance.

- 4. When the bulb is used, the change of power supply voltage should not exceed (+10%). If the voltage is too high, the life of the bulb will be shortened. If the voltage is too low, the light color of the bulb will be affected.

- 5. After power failure, it takes 5-10 minutes for the lamp to be fully cooled before it can be re-energized.

- 6. To ensure the normal use of this product, please read the instructions carefully.

- Product and Functional Effect Introduction:

- ·Using those that meet the international standards for using laser generators

- Semiconductor laser generator, laser source has long service life and high stability.

- Operation is very simple, humanized design, dimming without flicker, jitter and so on. It can be used in photography, photography, television and other places where lighting requirements are strict.

Classic effects: three-dimensional laser effects such as beam network, vast sea of clouds, 10,000 Zhang light, amplification and reduction, space-time tunnel, etc.Menu function:

Installation instructions for lamps:

The lamp can be placed horizontally, hanging diagonally and upside down, and you must pay attention to the installation method when hanging diagonally and upside down.

Fixing installation of lamps and lanterns: Before positioning the lamps and lanterns, ensure the stability of the installation site. When reversing the hanging installation, make sure that the lamps do not fall down on the support frame, and use safety ropes to pass through the support frame and the lamp handle. Carry out auxiliary hanging; to ensure safety. Prevent the lamp from falling and sliding. When the lamp is installed and debugged, pedestrians are prohibited from passing underneath. Regularly check whether the safety rope is worn and the hook screw is loose. If the hanging installation is unstable, the lamp will be caused The manufacturer does not bear any responsibility for all the consequences of the fall.

safety warning!

In order to ensure that you use the lamp safely and correctly, please read the following safety tips carefully before use to avoid unnecessary failures and injuries.

1.Non-professionals, do not disassemble the lamps and the accessories in the lamps without authorization.

2. AC power supply: Check whether the local power supply meets the product's rated voltage requirements.

3. This lamp is designed according to the type of electric shock protection. The lamp must be connected to a fully grounded power supply system, and the ground wire of the lamp must be connected to the ground wire of the power supply system. Do not use a power cord with a damaged insulation layer, and do not The power cord is connected to other wires.

4. When the lamp is installed and positioned, the minimum distance between any point on the surface of the lamp and any combustible and explosive objects is 10 meters, and the distance from the irradiated object is 2.5 meters. Please do not install the lamp directly on the surface of burnable materials.

5. The ambient temperature of the lamp: (-10 degrees + 40 degrees), the maximum temperature of the surface of the lamp is 80 degrees, and the lamp should be kept away from liquid substances and humid environments.

6.Before using the lamp, make sure that the lamp is well grounded, and do not install and remove any parts of the lamp with power.

7.When installing the lamp, the fixing screws must be tightened, with safety cables, and regular inspections.

8. It is recommended that the continuous working time of the lamp should not exceed 10 hours, and the interval between continuous starting of the lamp should not be less than 10 minutes. Otherwise, the lamp will not be triggered normally due to the overheating protection of the lamp.

9.During use, if the lamp is abnormal, stop using the lamp in time.

10.When the bulb reaches the rated service life, it should be replaced in time, otherwise there will be a dangerous accident of bubble explosion.

11.The rotating parts of the lamp and the pasted accessories must be checked regularly, and if there is looseness, the shaking should be reinforced in time to prevent accidents.

12.The lamp adopts strong wind cooling, which is easy to accumulate dust. It must be cleaned once a month, especially the heat dissipation vent, otherwise it will be blocked by the accumulation of dust, resulting in poor heat dissipation and abnormalities in the lamp.

- LCD screen laser menu and channel manual

- 1、 Main menu list

- Note 1: When the 25 pin ILDA port is connected to the computer software signal, the lamp automatically recognizes and switches to software control

- Note 2: If the lighting fixture is connected to computer software through a network port, please manually set it to network port mode in the mode settings of the menu (refer to the table below)

| 1 | Address code | 1-512 | Address code setting |
| --- | --- | --- | --- |
| 2 | Channel settings | 16CH/20CH | 16CH is the simplified channel mode, and 20CH is the professional channel mode |
| 3 | Mode setting |  | 0 DMX - Console Mode |
|  |  |  | 1 Ethernet port - switch to Ethernet port pangolin software control |
|  |  |  | 2 Random - All programs are random |
|  |  |  | 3 Lines - Line Program |
|  |  |  | 4 Animation - Animation Program |
|  |  |  | 5 Text - Play text programs (text input from mobile apps or mini programs) |
|  |  |  | 6 Programming - Play programming programs (programs are programmed by mobile apps or mini programs) |
|  |  |  | 7 Hand drawn - Play hand drawn patterns (patterns drawn by mobile apps or mini programs) |
|  |  |  | 8 Landmarks - Outdoor and Landmark Programs |
|  |  |  | 9 Standby - Lamp Off Light Standby |
| 4 | Play speed | 0 | Voice control |
|  |  | 1--99 | Self driving speed (from slow to fast) |
| 5 | Pattern Size | 10-100 | Pattern Size |
| 6 | Pattern phase | X+/Y+ | Pattern phase |
| 7 | Red light dimming | 0-100 | Red light dimming (effective analog modulation) |
| 8 | Green light dimming | 0-100 | Green light dimming (effective analog modulation) |
| 9 | Blue light dimming | 0-100 | Blue light dimming (effective analog modulation) |
| 10 | modulation mode | TTL/simulate | modulation mode |
| 11 | Language settings | Chinese/English | Language settings |
| 12 | Software version | VER2.X | VER2. X is the software version |

- Hidden menu (long press the menu button to enter the hidden menu, set it up, long press the menu button to exit)

| 1 | Mirror speed | 15-30K | Mirror speed setting (please set carefully to avoid exceeding the actual mirror speed) |
| --- | --- | --- | --- |
| 2 | color setting | Monochrome/full-color | Set the light source to monochrome or full-color |
| 3 | Single point protection | on/off | Single point protection for turning on and off laser galvanometer |
| 4 | Voice sensitivity | 0-100 | Voice sensitivity adjustment |
| 5 | Backlight Setting | Constant on/delayed off | Always on: The screen is always on. Delay off: 15 seconds to turn off the screen |
| 6 | Factory restoration | sure | Long press for 3 seconds to restore factory settings |

- 2、16Channel Mode

|  |  | DMX-512numerical value | control content |
| --- | --- | --- | --- |
| CH1 | Total dimming | 0-255 | Main switch/dimming (dimming function, effective analog modulation) |
| CH2 | Flicker | 0-9 | No flicker |
|  |  | 10-255 | The flicker speed changes from slow to fast |
| CH3 | colour | 0-34 | White red blue purple green (fixed color selection) |
|  |  | 35-39 | Overall color change (flow rate controlled by CH4) |
|  |  | 40-44 | Initial color of pattern (flow rate controlled by CH4) |
|  |  | 45-46 | Rainbow colors (flow rate controlled by CH4) |
|  |  | 47-66 | 2 segmented color selection, 4 numerical steps (flow rate controlled by CH4) |
|  |  | 67-96 | 3 segmented color selection, 4 numerical steps (flow rate controlled by CH4) |
|  |  | 97-126 | 4 segmented color selection, 4 numerical steps (flow rate controlled by CH4) |
|  |  | 127-150 | 8 segmented color selection, 4 numerical steps (flow rate controlled by CH4) |
|  |  | 151-174 | 16 segmented color selection with 4 numerical steps (flow rate controlled by CH4) |
|  |  | 175-214 | 32 segmented color selection with 4 numerical steps (flow rate controlled by CH4) |
|  |  | 215-247 | 32 types of simulated color mixing (effective modulation) |
|  |  | 248-255 | Color gradient painting (flow rate controlled by CH4) |
| CH4 | Color flow rate | 0-9 | Color does not flow smoothly |
|  |  | 10-127 | Color flowing forward, speed increasing from slow to fast |
|  |  | 128-255 | Color reversed flowing water, speed from slow to fast |
| CH5 | Graphic selection | 0-255 | Pattern selection (pattern grouping controlled by CH6) |
| CH6 | Graphic group selection | 0-24 | Built in static graphic group 1 (basic geometric patterns) |
|  |  | 25-49 | Built in static graphic group 2 (basic geometric patterns) |
|  |  | 50-74 | Built in static graphic group 3 (edge highlight pattern) |
|  |  | 75-99 | Built in static graphic group 4 (dotted graphic) |
|  |  | 100-124 | reserve |
|  |  | 125-149 | ILDA Animation Group 1 |
|  |  | 150-174 | ILDA Animation Group 2 |
|  |  | 175-199 | ILDA Animation Group 3 (temporarily retained) |
|  |  | 200-224 | ILDA Animation Group 4 |
|  |  | 225-255 | Hand drawn graphics (graphics edited by APP) |
| CH7 | Pattern Size | 0-255 | Manual selection of pattern size |
| CH8 | Automatic scaling of patterns | 0-15 | Pattern size selection |
|  |  | 16-55 | Speed selection from small to large |
|  |  | 56-95 | Speed selection from high to low |
|  |  | 96-135 | Size scaling speed selection |
|  |  | 136-175 | Two point irregular cyclic scaling |
|  |  | 176-215 | Three point irregular cyclic scaling |
|  |  | 216-255 | Four point irregular cyclic scaling |
| CH9 | The pattern rotates around the center | 0-127 | Rotation angle selection |
|  |  | 128-191 | Positive rotation speed selection |
|  |  | 192-255 | Reverse rotation speed selection |
| CH10 | The pattern rotates around the X-axis | 0-127 | Horizontal flip position selection |
|  |  | 128-255 | Horizontal Flip Speed Selection |
| CH11 | The pattern rotates around the Y-axis | 0-127 | Vertical Flip Position Selection |
|  |  | 128-255 | Vertical flipping speed selection |
| CH12 | horizontal movement | 0-255 | Horizontal position selection |
| CH13 | vertical movement | 0-255 | Vertical position selection |
| CH14 | X-direction waves | 0-1 | No waves |
|  |  | 2-255 | Wave amplitude and speed adjustment (amplitude from small to large, speed from slow to fast, every 32 values in one gear, a total of 8 gears) |
| CH15 | Built in dynamic effects | 0-1 | NO FUNCTION |
|  |  | 2-202 | Built in Effect Single Choice (Color CH3 Control, CH3=0 Default Color, Speed CH16 Control) |
|  |  | 203-214 | Line effect (color CH3 control, CH3=0 default color, speed CH16 control) |
|  |  | 215-224 | Animation effects (color CH3 control, CH3=0 default color, speed CH16 control) |
|  |  | 225-234 | Outdoor landmark effect (color CH3 control, CH3=0 default color, speed CH16 control) |
|  |  | 235-255 | All effects are randomly played (color CH3 control, CH3=0 default color, speed CH16 control) |
| CH16 | Built in dynamic effect speed | 0-1 | System default speed |
|  |  | 2-255 | Manually adjust the built-in effect speed from slow to fast |

- 3、20Channel Mode

|  |  | DMX-512numerical value | control content |
| --- | --- | --- | --- |
| CH1 | Total dimming | 0-255 | Main switch/dimming (dimming function, effective analog modulation) |
| CH2 | Red light dimming | 0-255 | Linear dimming of red light |
| CH3 | Green light dimming | 0-255 | Green linear dimming |
| CH4 | Blue light dimming | 0-255 | Blue light linear dimming |
| CH5 | Flicker | 0-9 | No flicker |
|  |  | 10-255 | The flicker speed changes from slow to fast |
| CH6 | colour | 0-34 | White red blue purple green (fixed color selection) |
|  |  | 35-39 | Overall color change (flow rate controlled by CH7) |
|  |  | 40-44 | Initial color of pattern (flow rate controlled by CH7) |
|  |  | 45-46 | Rainbow Colors (Flow rate controlled by CH7) |
|  |  | 47-66 | 2 segmented color selection, 4 numerical steps (flow rate controlled by CH7) |
|  |  | 67-96 | 3 segmented color selection, 4 numerical steps (flow rate controlled by CH7) |
|  |  | 97-126 | 4 segmented color selection, 4 numerical steps (flow rate controlled by CH7) |
|  |  | 127-150 | 8 segmented color selection, 4 numerical steps (flow rate controlled by CH7) |
|  |  | 151-174 | 16 segmented color selection with 4 numerical steps (flow rate controlled by CH7) |
|  |  | 175-214 | 32 segmented color selection with 4 numerical steps (flow rate controlled by CH7) |
|  |  | 215-247 | 32 types of simulated color mixing (effective modulation) |
|  |  | 248-255 | Color gradient painting (flow rate controlled by CH7) |
| CH7 | Color flow rate | 0-9 | Color does not flow smoothly |
|  |  | 10-127 | Color flowing forward, speed increasing from slow to fast |
|  |  | 128-255 | Color reversed flowing water, speed from slow to fast |
| CH8 | Graphic selection | 0-255 | Pattern selection (pattern grouping controlled by CH9) |
| CH9 | Graphic group selection | 0-24 | Built in static graphic group 1 (basic geometric patterns) |
|  |  | 25-49 | Built in static graphic group 2 (basic geometric patterns) |
|  |  | 50-74 | Built in static graphic group 3 (edge highlight pattern) |
|  |  | 75-99 | Built in static graphic group 4 (dotted graphic) |
|  |  | 100-124 | reserve |
|  |  | 125-149 | ILDA Animation Group 1 |
|  |  | 150-174 | ILDA Animation Group 2 |
|  |  | 175-199 | ILDA Animation Group 3 (temporarily retained) |
|  |  | 200-224 | ILDA Animation Group 4 |
|  |  | 225-255 | Hand drawn graphics (graphics edited by APP) |
| CH10 | Pattern Size | 0-255 | Manual selection of pattern size |
| CH11 | Automatic scaling of patterns | 0-15 | Pattern size selection |
|  |  | 16-55 | Speed selection from small to large |
|  |  | 56-95 | Speed selection from high to low |
|  |  | 96-135 | Size scaling speed selection |
|  |  | 136-175 | Two point irregular cyclic scaling |
|  |  | 176-215 | Three point irregular cyclic scaling |
|  |  | 216-255 | Four point irregular cyclic scaling |
| CH12 | The pattern rotates around the center | 0-127 | Rotation angle selection |
|  |  | 128-191 | Positive rotation speed selection |
|  |  | 192-255 | Reverse rotation speed selection |
| CH13 | The pattern rotates around the X-axis | 0-127 | Horizontal flip position selection |
|  |  | 128-255 | Horizontal Flip Speed Selection |
| CH14 | The pattern rotates around the Y-axis | 0-127 | Vertical Flip Position Selection |
|  |  | 128-255 | Vertical flipping speed selection |
| CH15 | horizontal movement | 0-255 | Horizontal position selection |
| CH16 | vertical movement | 0-255 | Vertical position selection |
| CH17 | X-direction waves | 0-1 | No waves |
|  |  | 2-255 | Wave amplitude and speed adjustment (amplitude from small to large, speed from slow to fast, every 32 values in one gear, a total of 8 gears) |
| CH18 | Pattern gradual drawing | 0-1 | No gradual drawing |
|  |  | 2-63 | Manual Gradient 1 |
|  |  | 64-127 | Manual Gradient Drawing 2 |
|  |  | 128-153 | Automatic Gradient (Increase) |
|  |  | 154-179 | Automatic Gradient (Subtraction) |
|  |  | 180-205 | Automatic gradient drawing (increase first, then decrease - reverse) |
|  |  | 206-255 | Automatic gradient drawing (increase first, then decrease - same direction) |
| CH19 | Built in dynamic effects | 0-1 | NO FUNCTION |
|  |  | 2-202 | Built in Effect Single Choice (Color CH3 Control, CH3=0 Default Color, Speed CH20 Control) |
|  |  | 203-214 | Line effect (color CH3 control, CH3=0 default color, speed CH20 control) |
|  |  | 215-224 | Animation effects (color CH3 control, CH3=0 default color, speed CH20 control) |
|  |  | 225-234 | Outdoor landmark effect (color CH3 control, CH3=0 default color, speed CH20 control) |
|  |  | 235-255 | All effects are randomly played (color CH3 control, CH3=0 default color, speed CH20 control) |
| CH20 | Built in dynamic effect speed | 0-1 | System default speed |
|  |  | 2-255 | Manually adjust the built-in effect speed from slow to fast |

Product warranty card

| Product Name | Product Model |
| --- | --- |

End users Unit name: Address: Contacts: Telephone:

| Date of manufacture | User Purchase Date |
| --- | --- |

Retailer Contacts: Telephone: Seal：

Note: The blanks mentioned above must be filled out and stamped truthfully by the end-users and sellers, otherwise they are not guaranteed.