DMX-30 Controller

- Front panel

- Manual button to select lights manually

- Delete to delete programs.

- ◄button, back-forward to previous step when editing.

- ►button, forward to next step.Add one more step automatically, after press this button. (with a dot on the fourth digital value)

- Chase button to edit or operating programs. Short press and indicator on to operate programs, press 3 seconds until indicator blink to edit program.

- CH1/CH6 channels fader

- 1-5 buttons, to select operating programs or select lights.

- Speed fader, it will be effected, when running programs.

- Fade fader, effected when running program only.

- Selecting lights manually

1、When power on indicator on, or press Manual button to enter manual mode.

- 2、Press number 1-6 to select lights(address).

- Program editing

- 1、Press Chase over 3 seconds, indicators blinks until shows up“----”to enter edit program.

- 2、Select programs, from number 1—6,totally 6 programs.（Max. steps: 99 step）

For example 1：when choose number 1, if number 1 is empty，it shows up“1---”.

For example 2：when choose number 2, if number 2 is empty，it shows up“2-01”.

- 3、Choose editing steps, use ◄and ►button to choose single editing steps, or to check the edited steps; press ►button to save previous step and enter next step.

- 4、Delete steps: Press Delete button 3 seconds until it blinks to delete the previous step.

- Exit editing: press Chase button until indicator status from blink to on.

- Program running

- 1、Press Chase button to enter running mode until it blinks and shows up“----”.

- 2、Use number 1—5 to choose programs.

- 3、Use Speed fader to adjust each steps stay time, stay time shortest on the top.

- 4、Fading: use Fade fader to adjust fading time between two steps, on the top fading time shortest

- 5、Program switch and exit, use number 1—5 to choose different programs, indicators on means with programs. Short press the button to exit playing mode and indicators off.

- V. Wireless introduction（wireless version only）

- 1、Star this device and on the switch

- 2、Press KEY button to adjust，

- 7 different colors for 7 code. The receiver and the controller match the same color, these two devices with be connected.

- 3、LED indicator instruction.

| Protocol | Color statue | Description |
| --- | --- | --- |
| Colors protocol | Red, green, blue, yellow, cyan, purple and white | 7 groups of wireless |
|  | Red flash | Sending DMX signal |
|  | Green flash | Receiving DMX signal |

- 4、instruction

1) press display button to sub-menu to set ID，

2) press ID setting again, add one more value of each press

3)ID code and corresponding LED colors：

1：RED---red；  2：GREEN--green；  3：RED+GREED--yellow；  4：BLUE--blue；

5：RED+BLUE----purple；	6：GREED+BLUE----cyan；   7：RED+GREED+BLUE---white